Infinite Rotation Maple Leaf Retro Light

1. Technical Specification

Voltage：AC100-240V50/60Hz

Power：250W

Light source：3*60W Amber LEDs，36*3W 3535 3IN1 LEDs，1*40W 4IN1 LEDs

Channel：International standard dmx512, 15/20/26CH selection

Control：DMX512, Master-slave,Auto, Sound mode

Dimmer：0-100％ linear dimmer

Strobe：25 times/sec

- DMX512 Channels description

a.15 Channels show

| Channel | Function | Value | Description |
| --- | --- | --- | --- |
| CH1 | X motor | 0-255 | 0-360 degree |
| CH2 | X motor infinite | 0-49 | 0-360 degree rotation, speed from fast to slow, X motor speed enabled 360 degree rotation |
|  |  | 50-152 | Clockwise infinite rotation, speed from fast to slow |
|  |  | 153-255 | Anti-Clockwise infinite rotation, speed from slow to fast |
| CH3 | Amber Master dimmer | 0-255 | Linear dimmer, from dark to bright |
| CH4 | Strobe | 0 | No function |
|  |  | 1-255 | Strobe, speed from slow to fast (1HZ-25HZ) |
| CH5 | Amber | 0-4 | Light off |
|  |  | 5-41 | Amber 1 |
|  |  | 42-76 | Amber 2 |
|  |  | 77-112 | Amber 3 |
|  |  | 113-148 | Amber 1 + Amber 2 |
|  |  | 149-184 | Amber 1 + Amber 3 |
|  |  | 185-220 | Amber 2 + Amber 3 |
|  |  | 221-255 | Amber 1 + Amber 2 + Amber 3 |
| CH6 | 3535RGB Master dimmer | 0-255 | Linear dimmer, from dark to bright |
| CH7 | Strobe | 0 | No function |
|  |  | 1-255 | Strobe, speed from slow to fast (1HZ-25HZ) |
| CH8 | 3535RGB 1 (Built-in effect running color) | 0-4 | Light off |
|  |  | 5-41 | Red |
|  |  | 42-76 | Green |
|  |  | 77-112 | Blue |
|  |  | 113-148 | Red + Green |
|  |  | 149-184 | Red + Blue |
|  |  | 185-220 | Green + Blue |
|  |  | 221-255 | Red + Green + Blue |
| CH9 | 3535RGB 2 (Built-in effect background) | 0-4 | Light off |
|  |  | 5-41 | Red |
|  |  | 42-76 | Green |
|  |  | 77-112 | Blue |
|  |  | 113-148 | Red + Green |
|  |  | 149-184 | Red + Blue |
|  |  | 185-220 | Green + Blue |
|  |  | 221-255 | Red + Green + Blue |
| CH10 | 3535RGB 3 | 0-4 | Light off |
|  |  | 5-41 | Red |
|  |  | 42-76 | Green |
|  |  | 77-112 | Blue |
|  |  | 113-148 | Red + Green |
|  |  | 149-184 | Red + Blue |
|  |  | 185-220 | Green + Blue |
|  |  | 221-255 | Red + Green + Blue |
| CH11 | Beam RGBW | 0-4 | Light off |
|  |  | 5-36 | Red |
|  |  | 37-48 | Green |
|  |  | 49-79 | Blue |
|  |  | 80-111 | White |
|  |  | 112-144 | Red + Green |
|  |  | 145-176 | Red + Blue |
|  |  | 177-208 | Green + Blue |
|  |  | 209-240 | Red + Green + Blue |
|  |  | 241-255 | Red + Green + Blue + White |
| CH12 | Amber Macro | 0-4 | No function |
|  |  | 5-85 | Built-in effect 1 (from slow to fast) |
|  |  | 86-170 | Built-in effect 2 (from slow to fast) |
|  |  | 171-255 | Built-in effect 3 (from slow to fast) |
| CH13 | 3535RGB Macro | 0-4 | No function |
|  |  | 5-45 | Built-in effect 1 (from slow to fast) |
|  |  | 46-87 | Built-in effect 2 (from slow to fast) |
|  |  | 88-129 | Built-in effect 3 (from slow to fast) |
|  |  | 130-171 | Built-in effect 4 (from slow to fast) |
|  |  | 172-213 | Built-in effect 5 (from slow to fast) |
|  |  | 214-255 | Built-in effect 6 (from slow to fast) |
| CH14 | Beam RGBW Macro | 0-4 | No function |
|  |  | 5-85 | Built-in effect 1 (from slow to fast) |
|  |  | 86-170 | Built-in effect 2 (from slow to fast) |
|  |  | 171-255 | Built-in effect 3 (from slow to fast) |
| CH15 | Reset | 0-199 | No function |
|  |  | 200-255 | Reset |

- 20 Channels show

| Channel | Function | Value | Description |
| --- | --- | --- | --- |
| CH1 | X motor | 0-255 | 0-360 degree |
| CH2 | X motor infinite | 0-49 | 0-360 degree rotation, speed from fast to slow, X motor speed enabled 360 degree rotation |
|  |  | 50-152 | Clockwise infinite rotation, speed from fast to slow |
|  |  | 153-255 | Anti-Clockwise infinite rotation, speed from slow to fast |
| CH3 | Amber Master dimmer | 0-255 | Linear dimmer, from dark to bright |
| CH4 | Strobe | 0 | No function |
|  |  | 1-255 | Strobe, speed from slow to fast (1HZ-25HZ) |
| CH5 | Amber 1 | 0-255 | Linear dimmer, from dark to bright |
| CH6 | Amber 2 | 0-255 | Linear dimmer, from dark to bright |
| CH7 | Amber 3 | 0-255 | Linear dimmer, from dark to bright |
| CH8 | 3535RGB Master dimmer | 0-255 | Linear dimmer, from dark to bright |
| CH9 | Strobe | 0 | No function |
|  |  | 1-255 | Strobe, speed from slow to fast (1HZ-25HZ) |
| CH10 | 3535RGB Red (Built-in effect running color) | 0-255 | Linear dimmer, from dark to bright |
| CH11 | 3535RGB Green (Built-in effect background) | 0-255 | Linear dimmer, from dark to bright |
| CH12 | 3535RGB Blue | 0-255 | Linear dimmer, from dark to bright |
| CH13 | Beam RGBW Red | 0-255 | Linear dimmer, from dark to bright |
| CH14 | Beam RGBW Green | 0-255 | Linear dimmer, from dark to bright |
| CH15 | Beam RGBW Blue | 0-255 | Linear dimmer, from dark to bright |
| CH16 | Beam RGBW White | 0-255 | Linear dimmer, from dark to bright |
| CH17 | Amber Macro | 0-4 | No function |
|  |  | 5-85 | Built-in effect 1 (from slow to fast) |
|  |  | 86-170 | Built-in effect 2 (from slow to fast) |
|  |  | 171-255 | Built-in effect 3 (from slow to fast) |
| CH18 | 3535RGB Macro | 0-4 | No function |
|  |  | 5-45 | Built-in effect 1 (from slow to fast) |
|  |  | 46-87 | Built-in effect 2 (from slow to fast) |
|  |  | 88-129 | Built-in effect 3 (from slow to fast) |
|  |  | 130-171 | Built-in effect 4 (from slow to fast) |
|  |  | 172-213 | Built-in effect 5 (from slow to fast) |
|  |  | 214-255 | Built-in effect 6 (from slow to fast) |
| CH19 | Beam RGBW Macro | 0-4 | No function |
|  |  | 5-85 | Built-in effect 1 (from slow to fast) |
|  |  | 86-170 | Built-in effect 2 (from slow to fast) |
|  |  | 171-255 | Built-in effect 3 (from slow to fast) |
| CH20 | Reset | 0-199 | No function |
|  |  | 200-255 | Reset |

- 26 Channels show

| Channel | Function | Value | Description |
| --- | --- | --- | --- |
| CH1 | X motor | 0-255 | 0-360 degree |
| CH2 | X motor infinite | 0-49 | 0-360 degree rotation, speed from fast to slow, X motor speed enabled 360 degree rotation |
|  |  | 50-152 | Clockwise infinite rotation, speed from fast to slow |
|  |  | 153-255 | Anti-Clockwise infinite rotation, speed from slow to fast |
| CH3 | Amber Master dimmer | 0-255 | Linear dimmer, from dark to bright |
| CH4 | Strobe | 0 | No function |
|  |  | 1-255 | Strobe, speed from slow to fast (1HZ-25HZ) |
| CH5 | Amber 1 | 0-255 | Linear dimmer, from dark to bright |
| CH6 | Amber 2 | 0-255 | Linear dimmer, from dark to bright |
| CH7 | Amber 3 | 0-255 | Linear dimmer, from dark to bright |
| CH8 | 3535RGB Master dimmer | 0-255 | Linear dimmer, from dark to bright |
| CH9 | Strobe | 0 | No function |
|  |  | 1-255 | Strobe, speed from slow to fast (1HZ-25HZ) |
| CH10 | 3535RGB Red 1 (Built-in effect running color) | 0-255 | Linear dimmer, from dark to bright |
| CH11 | 3535RGB Green 1 (Built-in effect background) | 0-255 | Linear dimmer, from dark to bright |
| CH12 | 3535RGB Blue 1 | 0-255 | Linear dimmer, from dark to bright |
| CH13 | 3535RGB Red 2 | 0-255 | Linear dimmer, from dark to bright |
| CH14 | 3535RGB Green 2 | 0-255 | Linear dimmer, from dark to bright |
| CH15 | 3535RGB Blue 2 | 0-255 | Linear dimmer, from dark to bright |
| CH16 | 3535RGB Red 3 | 0-255 | Linear dimmer, from dark to bright |
| CH17 | 3535RGB Green 3 | 0-255 | Linear dimmer, from dark to bright |
| CH18 | 3535RGB Blue 3 | 0-255 | Linear dimmer, from dark to bright |
| CH19 | Beam RGBW Red | 0-255 | Linear dimmer, from dark to bright |
| CH20 | Beam RGBW Green | 0-255 | Linear dimmer, from dark to bright |
| CH21 | Beam RGBW Blue | 0-255 | Linear dimmer, from dark to bright |
| CH22 | Beam RGBW White | 0-255 | Linear dimmer, from dark to bright |
| CH23 | Amber Macro | 0-4 | No function |
|  |  | 5-85 | Built-in effect 1 (from slow to fast) |
|  |  | 86-170 | Built-in effect 2 (from slow to fast) |
|  |  | 171-255 | Built-in effect 3 (from slow to fast) |
| CH24 | 3535RGB Macro | 0-4 | No function |
|  |  | 5-45 | Built-in effect 1 (from slow to fast) |
|  |  | 46-87 | Built-in effect 2 (from slow to fast) |
|  |  | 88-129 | Built-in effect 3 (from slow to fast) |
|  |  | 130-171 | Built-in effect 4 (from slow to fast) |
|  |  | 172-213 | Built-in effect 5 (from slow to fast) |
|  |  | 214-255 | Built-in effect 6 (from slow to fast) |
| CH25 | Beam RGBW Macro | 0-4 | No function |
|  |  | 5-85 | Built-in effect 1 (from slow to fast) |
|  |  | 86-170 | Built-in effect 2 (from slow to fast) |
|  |  | 171-255 | Built-in effect 3 (from slow to fast) |
| CH26 | Reset | 0-199 | No function |
|  |  | 200-255 | Reset |