- X760

User Manual

- Safety Instructions

- Please read the instruction carefully which includes important information about the installation, usage and maintenance.

- Please keep this User Guide for future consultation. If you sell the unit to another user, be sure that they also receive this instruction manual.

- Unpack and check carefully to ensure that there is no transportation damage before using the unit.

- This product is suitable for wet locations. Do not immerse in water.

- DO install and operate by qualified operator.

- DO NOT allow children to operate the fixture.

- Use safety chain when fixing the unit. Handle the unit by carrying its base instead of head only.

- The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent surfaces.

- Be sure that no ventilation slots is blocked, otherwise the unit will be overheated.

- Before operation, ensure that you are connecting this product to the proper voltage in accordance with the specifications in this manual or on the product’s specification label.

- It’s important to ground the yellow/green conductor to earth in order to avoid electric shock.

- Minimum ambient temperature TA: 0℃. Maximum ambient temperature TA: 40℃. Do not operate this product at a lower or higher temperature.

- DO NOT connect the device to any dimmer pack

- Keep flammable materials away from the fixture while operating to avoid fire hazard.

- Make sure the power cord is not crimped or damaged; replace it immediately if damaged

- Unit’s surface temperature may reach up to 65℃. DO NOT touch the housing bare-handed during its operation

- Avoid any flammable liquids, water or metal from entering the unit.

- DO NOT operate in a dirty or dusty environment. DO clean the fixture regularly.

- DO NOT touch any wire during operation as there might be a hazard of electric shock.

- Avoid entanglement of the power cord with other wires.

- The minimum distance to objects/surface must be more than 0.5 meters.

- In the event of serious operating problem, stop using the unit immediately.

- Never turn on and off the unit time after time.

- The housing, the lenses, or the ultraviolet filter must be replaced if they are visibly damaged

- DO NOT open the housing as there are no user serviceable parts inside.

- DO NOT attempt to operate this unit if it becomes damaged. DO NOT attempt any repairs yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please contact the nearest authorized technical assistance center if needed.

- Disconnect this product from its power source before servicing.

- DO use the original packaging if the device is to be transported.

- Avoid direct eye exposure to the light source while the product is on.

- DO NOT operate this product if you see damage on the housing, shields, or cables. Have the damaged parts replaced by an authorized technician at once.

2.INSTALLATIONS

- installations Note: In order to increase protection, please install the lamp on the sidewalk, outside the seating area, or an area where unauthorized persons may touch the led

- Before installing the fixture on any surface, make sure that the installation area can bear the minimum point load above 10 points of the weight of the equipment. The installation of the fixing device must always be fixed with auxiliary safety accessories (such as a suitable safety rope) Do not stand directly under the equipment when installing, removing, or servicing fixtures

- Ensure thatthis fixture is kept at least 0.5m (1.5 feet) away from any

flammable materials(decorations, etc.)

- Be sure to use and install the supplied safety rope to ensure safety and

prevent accidental damage and/or injury in case the fixture is damaged

Installation point: Overhead installation requires a wealth of experience,

including calculation of working load limits, in-depth understanding of the

installation materials used, and regular safety inspections of all installation

materials and fixtures. If you do not have these qualifications, please do not try to install it yourself. Improper installation can cause personal injury

Before connecting the main power cord to an appropriate wall outlet,make

sure to complete all assembly and installation procedures

3. DMX-512 control connections

Connect the provided XLR cable to the female 3-pin XLR output of your

controller and the other side to the male 3-pin XLR input of the moving head.

You can chain multiple Moving heads be connected together through serial linking. The cable needed should be two core, screened cable with XLR input and output connectors.

For installations where the DMX cable has to run a long distance or is in an

electrically noisy environment, such as in a discotheque, it is recommended to

use a DMX terminator. This helps in preventing corruption of the digital control

signal by electrical noise. The DMX terminator is simply an XLR plug with a 120 Ω resistor connected between pins 2 and 3,which is then plugged into the

output XLR socket of the last fixture in the chain. Please see illustrations below

4.Address code setting

If you use a universal DMX controller to control the units, you have to set DMX address from 1 to 512 so that the units can receive DMX signal.

Press the MENU button to enter menu mode, select DMX Settings, press the ENTER button to confirm, use the UP/DOWN button to select DMX Address, press the ENTER button to confirm, the present address will blink in the display, use the UP/DOWN button to adjust the address from 001 to 512, press the ENTER button to store. Press the MENU button back to the last menu or let the unit idle 30 seconds to exit menu mode.Please refer to the following diagram to address your DMX512 channel for the first 4 units.

| Channel MODE | UNIT1 ADDERSS | UNIT2 ADDERSS | UNIT3 ADDERSS | UNIT4 ADDERSS |
| --- | --- | --- | --- | --- |
| 20CH | 1.1 | 1.21 | 1.41 | 1.61 |
| 36CH | 1.1 | 1.37 | 1.73 | 1.109 |
| 57CH | 1.1 | 1.58 | 1.115 | 1.172 |
| 66CH | 1.1 | 1.67 | 1.133 | 1.199 |
| 93CH | 1.1 | 1.94 | 1.187 | 1.280 |
| 102CH | 1.1 | 1.103 | 1.205 | 1.307 |
| 214CH | 1.1 | 1.215 | 2.1 | 2.215 |

- Fixture size

- LED PIXEL ZONES

- 57/66CH pixel segment

- 93/102CH pixel segment

- 214CH pixel segment

7.LED technical parameters

Input voltage: 100-240V, 50/60Hz

Power consumption: 520W

Power supply: 600W

Main light source: 7PCSx60W RGBW 4IN 1

Auxiliary light 180PCSX0.2W 3535RGB 3IN1

Color System: Excellent Color Macro Effects

Powerful color mixing effect

Rich dynamic color effects

Pixel matrix: Main light 7psc controlled separately

Auxiliary light 12 segment independent control

It can also be controlled by a group of three light beads for point control

LED lifespan:>20000 hours

Zoom angle: 4-60 °

CTO:2500k-8000k

DMX channel: 20/36/57/93/102/214 channels

Operation mode: DMX512、 Automatic, master-slave/ RDM

Dimmer: 0-100% smooth linear dimming

Dimming curves: linear, square law, inverse square law, S-curve

Dimming speed: fast, slow

Strobe: 1-25HZ strong output flicker effect

Adjustable refresh rate from 1K-64KHz

Display: OLED display

Signal input/output: 3-pin and 5-pin waterproof XLR sockets

Power socket: Waterproof power connector input/output

Protection level: IP65

Size: 228x332x435mm

Net weight: 14kg

Feature

The fixture is composed of 7 60W RGBW four-color LEDs as the light source, with excellent color macro effects, making it easy to create powerful beams. Multiple channel modes are available and the LED can be controlled point by point, allowing for flexible color switching. A linear focusing system with a zoom angle of 5 ° -50 °, multiple speed strobe effects, and smooth dimming function from 0 to 100%. Adjustable refresh rate from 1K-64KHz，The backlight is composed of 180 0.2W 3535RGB LEDs, which can create a variety of dynamic lighting effects. The LED can be controlled point by point or divided into 12 stages, fully adapting to different stage application needs..Not only is it compact and lightweight,but it’s X and Y axes also operate with exceptional smoothness and minimal noise

8.Menu Description

| Main Menu | First level menu | Secondary menu | Third level menu |
| --- | --- | --- | --- |
| DMX Address | 001-255 |  |  |
| Channel Mode | 20/36/57/66/93/ 102/214 |  |  |
| Run | DMX |  |  |
|  | Auto |  |  |
| Reset motor | Reset All |  |  |
|  | Pan/tilt |  |  |
|  | zoom |  |  |
|  | Return |  |  |
| settings | dimmer |  |  |
|  | Master /remote | Master/slave |  |
|  | Dimmer curve | square |  |
|  |  | linear |  |
|  |  | Square Root |  |
|  |  | S-Curve |  |
|  | Pan Reverse | N/Y |  |
|  | Dim speed | Fast/slow |  |
|  | Dim frequency | 1KHZ-64KHZ |  |
|  | Pan Reverse | N/Y |  |
|  | Tilt Reverse | N/Y |  |
|  | Pan Range | Start | 000 |
|  |  | End | 255 |
|  | Tilt Range | Start | 000 |
|  |  | End | 255 |
| z | Fan Control | Auto |  |
|  |  | high |  |
|  |  | medium |  |
|  |  | low |  |
|  |  | Ultra low |  |
|  | Lost signal | Keep/dim off |  |
|  | return |  |  |
| DMX Manual | pan | 0-255 |  |
|  | tilt | 0-255 |  |
|  | .... | 0-255 |  |
|  | ... | 0-255 |  |
|  | return |  |  |
|  | Fine Adj | Password:xxx |  |
| Adv.settings | Fine Adj Assist | Recover |  |
|  |  | Show backup |  |
|  | Display set | sleep | 30sec/... |
|  |  | Invert | N/Y |
|  |  | Display lock | On/off |
|  |  | Display | On/off |
|  | language | Eng/中文 |  |
|  | Recover set | On/off |  |
|  | Return |  |  |
| Factory set | Password:xxx |  |  |
| Info | Dmx monitor | Dmx value | 001-032 |
|  |  |  | ..... |
|  |  | Dmx value | 257-282 |
|  |  | FPS | 000 |
|  |  | Total chns | 000 |
|  |  | return |  |
|  | Test | Password:xxx |  |
|  | LED Usage time | Current | Xxxx min |
|  |  | Total | Xxx hours |
|  |  | clear |  |
|  | Error | xxxx |  |
|  | Vers | xxxx |  |

- DMX Channel

- 20Channel

| 20CH | DMX | Explanation |
| --- | --- | --- |
| 1 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 2 | 0-255 | dimmer |
| 3 | 0-255 | Dimmer fine |
| 4 | 0-255 | Red |
| 5 | 0-255 | Red fine |
| 6 | 0-255 | Green |
| 7 | 0-255 | Green fine |
| 8 | 0-255 | Blue |
| 9 | 0-255 | Blue fine |
| 10 | 0-255 | CTC |
| 11 |  | NO function |
| 12 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 13 | 0-255 | Zoom |
| 14 | 0-255 | Zoom fine |
| 15 | 0-255 | pan |
| 16 | 0-255 | Pan fine |
| 17 | 0-255 | tilt |
| 18 | 0-255 | Tilt fine |
| 19 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Screen Lock：YES |
|  | 53 | Screen Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |
| 20 |  | LED frequency |
|  | 0-9 | No function |
|  | 10 | 1KHz LED frequency |
|  | 11 | 2KHz LED frequency |
|  | 12 | 3KHz LED frequency |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 70 | 61KHz LED frequency |
|  | 71 | 62KHz LED frequency |
|  | 72 | 63KHz LED frequency |
|  | 73-127 | No function |
|  | 128 | 16KHz LED frequency（default） |
|  | 129-254 | No function |
|  | 255 | 64KHz LED frequency |

- 36Channel

| 36CH | DMX | Explanation |
| --- | --- | --- |
| 1 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 2 | 0-255 | dimmer |
| 3 | 0-255 | Dimmer fine |
| 4 | 0-255 | Red |
| 5 | 0-255 | Red fine |
| 6 | 0-255 | Green |
| 7 | 0-255 | Green fine |
| 8 | 0-255 | Blue |
| 9 | 0-255 | Blue fine |
| 10 | 0-255 | CTC |
| 11 |  | NO function |
| 12 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 13 | 0-255 | Zoom |
| 14 | 0-255 | Zoom fine |
| 15 | 0-255 | pan |
| 16 | 0-255 | Pan fine |
| 17 | 0-255 | tilt |
| 18 | 0-255 | Tilt fine |
| 19 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Dis Lock：YES |
|  | 53 | dis Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |
| 20 |  | LED frequency |
|  | 0-9 | No function |
|  | 10 | 1KHz LED frequency |
|  | 11 | 2KHz LED frequency |
|  | 12 | 3KHz LED frequency |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 70 | 61KHz LED frequency |
|  | 71 | 62KHz LED frequency |
|  | 72 | 63KHz LED frequency |
|  | 73-127 | No function |
|  | 128 | 16KHz LED frequency（default） |
|  | 129-254 | No function |
|  | 255 | 64KHz LED frequency |
| 21 |  | NO function |
| 22 |  | Main LED effect selection |
| 23 |  | Main LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 130- 255 | Effect forward slow → fast |
| 24 |  | Backlight LED effect selection |
| 25 |  | Backlight LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 130- 255 | Effect forward slow → fast |
| 26 |  | Effect Offset shift |
|  | 0 | NO Function |
|  | 1 | Offset shift10° |
|  | 2 | Offset shift20° |
|  | 3 | Offset shift30° |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 33 | Offset shift330° |
|  | 34 | Offset shift340° |
|  | 35 | Offset shift350° |
|  | 36 | Synchronized: all fixtures start FX cycles at same time |
|  | 37 - 100 | No function |
|  | 101- 120 | Random start (Channel 23 controls overall speed) |
|  | 121- 140 | Random duration |
|  | 141- 255 | No function |
| 27 |  | Backlight LED strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 28 | 0-255 | dimmer |
| 29 | 0-255 | Dimmer fine |
| 30 | 0-255 | Red |
| 31 | 0-255 | Green |
| 32 | 0-255 | blue |
| 33 | 0-255 | CTC |
| 34 | 0-255 | NO function |
| 35 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 36 |  | No function |

- 57Channel

| 57CH | DMX | Explanation |
| --- | --- | --- |
| 1 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 2 | 0-255 | dimmer |
| 3 | 0-255 | Dimmer fine |
| 4 | 0-255 | Red |
| 5 | 0-255 | Red fine |
| 6 | 0-255 | Green |
| 7 | 0-255 | Green fine |
| 8 | 0-255 | Blue |
| 9 | 0-255 | Blue fine |
| 10 | 0-255 | CTC |
| 11 |  | NO function |
| 12 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 13 | 0-255 | Zoom |
| 14 | 0-255 | Zoom fine |
| 15 | 0-255 | pan |
| 16 | 0-255 | Pan fine |
| 17 | 0-255 | tilt |
| 18 | 0-255 | Tilt fine |
| 19 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Screen Lock：YES |
|  | 53 | Screen Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |
| 20 |  | LED frequency |
|  | 0-9 | No function |
|  | 10 | 1KHz LED frequency |
|  | 11 | 2KHz LED frequency |
|  | 12 | 3KHz LED frequency |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 70 | 61KHz LED frequency |
|  | 71 | 62KHz LED frequency |
|  | 72 | 63KHz LED frequency |
|  | 73-127 | No function |
|  | 128 | 16KHz LED frequency（default） |
|  | 129-254 | No function |
|  | 255 | 64KHz LED frequency |
| 21 |  | NO function |
| 22 |  | Main LED effect selection |
| 23 |  | Main LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 24 |  | Backlight LED effect selection |
| 25 |  | Backlight LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 26 |  | Effect Offset shift |
|  | 0 | NO Function |
|  | 1 | Offset shift10° |
|  | 2 | Offset shift20° |
|  | 3 | Offset shift30° |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 33 | Offset shift330° |
|  | 34 | Offset shift340° |
|  | 35 | Offset shift350° |
|  | 36 | Synchronized: all fixtures start FX cycles at same time |
|  | 37 - 100 | No function |
|  | 101- 120 | Random start (Channel 23 controls overall speed) |
|  | 121- 140 | Random duration |
|  | 141- 255 | No function |
| 27 |  | Backlight LED strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 28 | 0-255 | dimmer |
| 29 | 0-255 | Dimmer fine |
| 30 | 0-255 | Red |
| 31 | 0-255 | Green |
| 32 | 0-255 | blue |
| 33 | 0-255 | CTC |
| 34 | 0-255 | NO function |
| 35 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 36 |  | No function |
| 37 | 0-255 | Pixel 1 Red |
| 38 | 0-255 | Pixel 1 green |
| 39 | 0-255 | Pixel 1 blue |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 55 | 0-255 | Pixel 7 Red |
| 56 | 0-255 | Pixel 7 green |
| 57 | 0-255 | Pixel 7 blue |

66Channel

| 66CH | DMX | Explanation |
| --- | --- | --- |
| 1 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 2 | 0-255 | dimmer |
| 3 | 0-255 | Dimmer fine |
| 4 | 0-255 | Red |
| 5 | 0-255 | Red fine |
| 6 | 0-255 | Green |
| 7 | 0-255 | Green fine |
| 8 | 0-255 | Blue |
| 9 | 0-255 | Blue fine |
| 10 | 0-255 | white |
| 11 | 0-255 | White fine |
| 12 | 0-255 | CTC |
| 13 |  | NO function |
| 14 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 15 | 0-255 | Zoom |
| 16 | 0-255 | Zoom fine |
| 17 | 0-255 | pan |
| 18 | 0-255 | Pan fine |
| 19 | 0-255 | tilt |
| 20 | 0-255 | Tilt fine |
| 21 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Screen Lock：YES |
|  | 53 | Screen Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |
| 22 |  | LED frequency |
|  | 0-9 | No function |
|  | 10 | 1KHz LED frequency |
|  | 11 | 2KHz LED frequency |
|  | 12 | 3KHz LED frequency |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 70 | 61KHz LED frequency |
|  | 71 | 62KHz LED frequency |
|  | 72 | 63KHz LED frequency |
|  | 73-127 | No function |
|  | 128 | 16KHz LED frequency（default） |
|  | 129-254 | No function |
|  | 255 | 63KHz LED frequency |
| 23 |  | NO function |
| 24 |  | Main LED effect selection |
| 25 |  | Main LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 26 |  | Backlight LED effect selection |
| 27 |  | Backlight LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 28 |  | Effect Offset shift |
|  | 0 | NO Function |
|  | 1 | Offset shift10° |
|  | 2 | Offset shift20° |
|  | 3 | Offset shift30° |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 33 | Offset shift330° |
|  | 34 | Offset shift340° |
|  | 35 | Offset shift350° |
|  | 36 | Synchronized: all fixtures start FX cycles at same time |
|  | 37-100 | No function |
|  | 101-120 | Random start (Channel 23 controls overall speed) |
|  | 121-140 | Random duration |
|  | 141-255 | No function |
| 29 |  | Backlight LED strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 30 | 0-255 | dimmer |
| 31 | 0-255 | Dimmer fine |
| 32 | 0-255 | Red |
| 33 | 0-255 | Green |
| 34 | 0-255 | blue |
| 35 | 0-255 | CTC |
| 36 | 0-255 | NO function |
| 37 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 38 |  | No function |
| 39 | 0-255 | Pixel 1 Red |
| 40 | 0-255 | Pixel 1 green |
| 41 | 0-255 | Pixel 1 blue |
| 42 | 0-255 | Pixel 1 white |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 63 | 0-255 | Pixel 7 Red |
| 64 | 0-255 | Pixel 7 green |
| 65 | 0-255 | Pixel 7 blue |
| 66 | 0-255 | Pixel 7 white |

93Channel

| 93CH | DMX | Explanation |
| --- | --- | --- |
| 1 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 2 | 0-255 | dimmer |
| 3 | 0-255 | Dimmer fine |
| 4 | 0-255 | Red |
| 5 | 0-255 | Red fine |
| 6 | 0-255 | Green |
| 7 | 0-255 | Green fine |
| 8 | 0-255 | Blue |
| 9 | 0-255 | Blue fine |
| 10 | 0-255 | CTC |
| 11 |  | NO function |
| 12 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 13 | 0-255 | Zoom |
| 14 | 0-255 | Zoom fine |
| 15 | 0-255 | pan |
| 16 | 0-255 | Pan fine |
| 17 | 0-255 | tilt |
| 18 | 0-255 | Tilt fine |
| 19 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Screen Lock：YES |
|  | 53 | Screen Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |
| 20 |  | LED frequency |
|  | 0-9 | No function |
|  | 10 | 1KHz LED frequency |
|  | 11 | 2KHz LED frequency |
|  | 12 | 3KHz LED frequency |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 70 | 61KHz LED frequency |
|  | 71 | 62KHz LED frequency |
|  | 72 | 63KHz LED frequency |
|  | 73-127 | No function |
|  | 128 | 16KHz LED frequency（default） |
|  | 129-254 | No function |
|  | 255 | 63KHz LED frequency |
| 21 |  | NO function |
| 22 |  | Main LED effect selection |
| 23 |  | Main LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 24 |  | Backlight LED effect selection |
| 25 |  | Backlight LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 26 |  | Effect Offset shift |
|  | 0 | NO Function |
|  | 1 | Offset shift10° |
|  | 2 | Offset shift20° |
|  | 3 | Offset shift30° |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 33 | Offset shift330° |
|  | 34 | Offset shift340° |
|  | 35 | Offset shift350° |
|  | 36 | Synchronized: all fixtures start FX cycles at same time |
|  | 37-100 | No function |
|  | 101-120 | Random start (Channel 23 controls overall speed) |
|  | 121-140 | Random duration |
|  | 141-255 | No function |
| 27 |  | Backlight LED strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 28 | 0-255 | dimmer |
| 29 | 0-255 | Dimmer fine |
| 30 | 0-255 | Red |
| 31 | 0-255 | Green |
| 32 | 0-255 | blue |
| 33 | 0-255 | CTC |
| 34 | 0-255 | NO function |
| 35 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 36 |  | No function |
| 37 | 0-255 | Pixel 1 Red |
| 38 | 0-255 | Pixel 1 green |
| 39 | 0-255 | Pixel 1 blue |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 55 | 0-255 | Pixel 7 Red |
| 56 | 0-255 | Pixel 7 green |
| 57 | 0-255 | Pixel 7 blue |
| 58 | 0-255 | Back light 1Red |
| 59 | 0-255 | Back light 1 green |
| 60 | 0-255 | Back light 1 blue |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 91 | 0-255 | Back light 12 Red |
| 92 | 0-255 | Back light 12 green |
| 93 | 0-255 | Back light 12 blue |

102Channel

| 102CH | DMX | Explanation |
| --- | --- | --- |
| 1 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 2 | 0-255 | dimmer |
| 3 | 0-255 | Dimmer fine |
| 4 | 0-255 | Red |
| 5 | 0-255 | Red fine |
| 6 | 0-255 | Green |
| 7 | 0-255 | Green fine |
| 8 | 0-255 | Blue |
| 9 | 0-255 | Blue fine |
| 10 | 0-255 | white |
| 11 | 0-255 | White fine |
| 12 | 0-255 | CTC |
| 13 |  | NO function |
| 14 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 15 | 0-255 | Zoom |
| 16 | 0-255 | Zoom fine |
| 17 | 0-255 | pan |
| 18 | 0-255 | Pan fine |
| 19 | 0-255 | tilt |
| 20 | 0-255 | Tilt fine |
| 21 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Screen Lock：YES |
|  | 53 | Screen Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |
| 22 |  | LED frequency |
|  | 0-9 | No function |
|  | 10 | 1KHz LED frequency |
|  | 11 | 2KHz LED frequency |
|  | 12 | 3KHz LED frequency |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 70 | 61KHz LED frequency |
|  | 71 | 62KHz LED frequency |
|  | 72 | 63KHz LED frequency |
|  | 73-127 | No function |
|  | 128 | 16KHz LED frequency（default） |
|  | 129-254 | No function |
|  | 255 | 63KHz LED frequency |
| 23 |  | NO function |
| 24 |  | Main LED effect selection |
| 25 |  | Main LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 26 |  | Backlight LED effect selection |
| 27 |  | Backlight LED Effect speed |
|  | 0 - 126 | Effect reversed fast → slow |
|  | 127- 129 | Effect stops |
|  | 13 - 255 | Effect forward slow → fast |
| 28 |  | Effect Offset shift |
|  | 0 | NO Function |
|  | 1 | Offset shift10° |
|  | 2 | Offset shift20° |
|  | 3 | Offset shift30° |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 。。。 | 。。。 |
|  | 33 | Offset shift330° |
|  | 34 | Offset shift340° |
|  | 35 | Offset shift350° |
|  | 36 | Synchronized: all fixtures start FX cycles at same time |
|  | 37-100 | No function |
|  | 101-120 | Random start (Channel 23 controls overall speed) |
|  | 121-140 | Random duration |
|  | 141-255 | No function |
| 29 |  | Backlight LED strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 30 | 0-255 | dimmer |
| 31 | 0-255 | Dimmer fine |
| 32 | 0-255 | Red |
| 33 | 0-255 | Green |
| 34 | 0-255 | blue |
| 35 | 0-255 | CTC |
| 36 | 0-255 | NO function |
| 37 |  | Color macro |
|  | 0 -10 | Open |
|  | 11 | Moroccan pink (Lee 790) |
|  | 13 | Pink (Lee 157) |
|  | 15 | Special rose pink (Lee 332) |
|  | 17 | Follies pink (Lee 328) |
|  | 19 | Fuchsia pink (Lee 345) |
|  | 21 | Surprise pink (Lee 194) |
|  | 23 | Congo blue (Lee 181) |
|  | 25 | Tokyo blue (Lee 071) |
|  | 27 | Deep blue (Lee 120) |
|  | 29 | Just blue (Lee 079) |
|  | 31 | Medium blue (Lee 132) |
|  | 33 | Double CT blue (Lee 200) |
|  | 35 | Slate blue (Lee 161) |
|  | 37 | Full CT blue (Lee 201) |
|  | 39 | Half CT blue (Lee 202) |
|  | 41 | Steel blue (Lee 117) |
|  | 43 | Lighter blue (Lee 353) |
|  | 45 | Light blue (Lee 118) |
|  | 47 | Medium blue green (Lee 116) |
|  | 49 | Dark green (Lee 124) |
|  | 51 | Primary green (Lee 139) |
|  | 53 | Moss green (Lee 089) |
|  | 55 | Fern green (Lee 122) |
|  | 57 | Jas green (Lee 738) |
|  | 59 | Lime green (Lee 088) |
|  | 61 | Spring yellow (Lee 100) |
|  | 63 | Deep amber (Lee 104) |
|  | 65 | Chrome orange (Lee 179) |
|  | 67 | Orange (Lee 105) |
|  | 69 | Gold amber (Lee 021) |
|  | 71 | Millennium gold (Lee 778) |
|  | 73 | Deep golden amber (Lee 135) |
|  | 75 | Flame red (Lee 164) |
|  | 77 | Red magenta |
|  | 79 | Medium lavender |
|  | 81 | Pure white |
|  | 83 | Pure red |
|  | 85 | Pure yellow |
|  | 87 | Pure green |
|  | 89 | Pure cyan |
|  | 91 | Pure blue |
|  | 93 | Pure magenta |
|  | 95 | Peacock blue (LEE 115) |
|  | 97 | Dark lavender (LEE 180) |
|  | 99 | Double CT orange (LEE 287) |
|  | 101 | Full CT orange (LEE 204) |
|  | 103 | Half CT orange (LEE 205) |
|  | 105 | Deep Straw (LEE 015) |
|  | 107-190 | No function |
|  |  | Continuous rotation |
|  | 191 -214 | CW, fast → slow |
|  | 215 -219 | Stop |
|  | 220 -243 | CCW slow → fast |
|  |  | Random colors |
|  | 244 -247 | Fast |
|  | 248 -251 | Medium |
|  | 252 -255 | Slow |
| 38 |  | No function |
| 39 | 0-255 | Pixel 1 Red |
| 40 | 0-255 | Pixel 1 green |
| 41 | 0-255 | Pixel 1 blue |
| 42 | 0-255 | Pixel 1 white |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 63 | 0-255 | Pixel 7 Red |
| 64 | 0-255 | Pixel 7 green |
| 65 | 0-255 | Pixel 7 blue |
| 66 | 0-255 | Pixel 7 white |
| 67 | 0-255 | Back light 1Red |
| 68 | 0-255 | Back light 1 green |
| 69 | 0-255 | Back light 1 blue |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 100 | 0-255 | Back light 12Red |
| 101 | 0-255 | Back light 12 green |
| 102 | 0-255 | Back light 12 blue |

214Channel

| 102CH | DMX | Explanation |
| --- | --- | --- |
| 1 | 0-255 | X |
| 2 | 0-255 | Y |
| 3 | 0-255 | ZOOM |
| 4 |  | Strobe |
|  | 0 - 19 | closed |
|  | 20 - 49 | open |
|  | 50 - 200 | Strobe, slow from fast |
|  | 201- 210 | Shutter open |
|  | 211- 255 | Random strobe, slow from fast |
| 5 | 0-255 | dimmer |
| 6 | 0-255 | Red 1 |
| 7 | 0-255 | Green 1 |
| 8 | 0-255 | Blue 1 |
| 9 | 0-255 | White 1 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 30 | 0-255 | Red 7 |
| 31 | 0-255 | Green 7 |
| 32 | 0-255 | Blue 7 |
| 33 | 0-255 | White 7 |
| 34 | 0-255 | Back light Red 1 |
| 35 | 0-255 | Back light Green 1 |
| 36 | 0-255 | Back light Blue 1 |
| 37 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 | 。。。 |
| 。。 | 0-255 |  |
| 211 | 0-255 | Back light Red 60 |
| 212 | 0-255 | Back light Green 60 |
| 213 | 0-255 | Back light Blue 60 |
| 214 |  | function |
|  | 0-9 | No function |
|  | 10 - 14 | Reset all |
|  | 15 - 16 | No function |
|  | 17 | Reset beam |
|  | 18 | Reset XY |
|  | 19 - 22 | No function |
|  | 23 | Linear dimming curve |
|  | 24 | Square law dimming curve |
|  | 25 | Inverse square law dimming curve |
|  | 26 | S-curve dimming curve |
|  | 27-51 | No function |
|  | 52 | Screen Lock：YES |
|  | 53 | Screen Lock：NO |
|  | 54 | Auto fan speed |
|  | 55 | Full fan speed |
|  | 56 | Medium fan speed |
|  | 57 | Low fan speed |
|  | 58 | Ultra-low fan speed |
|  | 59-60 | No function |
|  | 61 | Dimmer speed:fast |
|  | 62 | Dimmer speed:slow |
|  | 63-64 | No function |
|  | 65 | Signal clear |
|  | 66 | Signal keep |
|  | 67-255 | No function |

- 10.Trouble shooting

- Following are a few common problems that may occur during operation. Here are some suggestions for easy troubleshooting:

- The unit does not work, no light and the fan does not work

- Check the connection of power and main fuse.

- Measure the mains voltage on the main connector.

- Check the power on LED.

- Not responding to DMX controller

- DMX LED should be on. If not, check DMX connectors, cables to see if link properly.

- If the DMX LED is on and no response to the channel, check the address settings and DMX polarity.

- If you have intermittent DMX signal problems, check the pins on connectors or on PCB of the unit or the previous one.

- Try to use another DMX controller.

- Check if the DMX cables run near or run alongside to high voltage cables that may cause damage or interference to DMX interface circuit.

- One of the channels is not working well

- The stepper motor might be damaged or the cable connected  to  the PCB is broken.