Outdoor Wireless LED Par Light

12X18W 6in1 LED

User manual

Please read the instructions carefully before use

Statement

The product has well capability and intact packing when leave factory. All of the user should comply with warning item and manual, any misuse cause of the damages are not  included  in our guarantee, and also can not be responsible for any malfunction & problem owing to ignore the manual.

1. Safety Instructions

* Please keep this User Guide for future consultation. If you sell the unit to another user, be sure that they also receive this instruction booklet.

* Unpack and check carefully there is no transportation damage before using the unit.

* Before  operating, ensure that the voltage and frequency of  power supply match

the power requirements of the unit.

* It’s  important  to  ground  the  yellow/green  conductor  to  earth  in  order to  avoid electric shock.

* The  unit  must  be  installed  in  a  location  with  adequate  ventilation,  at  least  50cm  from adjacent surfaces. Be sure that no ventilation slots are blocked.

Disconnect main power before replacement or servicing.

* Make sure there are no flammable materials close to the unit while operating as it is fire hazard.

* Use safety cable when fixes this unit. DO NOT handle the unit by taking its head only, but always by taking its base.

* Maximum ambient temperature is Ta: 40℃. DO NOT operate it where the temperature is higher than this Unit surface temperature may reach up to 85℃. DO NOT touch the housing bare-hand during its operation. Turn off the power and allow about 15 minutes for the unit to cool down before replacing or serving.

* In the event of serious operating problem, stop using the unit immediately.Never try to repair the unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please contact the nearest authorized technical assistance center. Always use the same type spare parts.

* DO NOT touch any wire during operation as high voltage might be causing electric shock.

Warning:

* Please set one light only as master when used Master-slave model in the same channel.

* DO NOT set any light as master when used DMX hold model.

* To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or

moisture.

* DO NOT open the unit within five minutes after switching off.

* The housing, the  lenses, or  the  ultraviolet filter must be replaced if they are visibly damaged.

Caution:

There are no user serviceable parts inside the unit. DO NOT open the housing or attempt any repairs yourself. In the unlikely event your unit may require service, please contact your nearest dealer.

Installation:

The unit should be mounted via its screw holes on the bracket. Always ensure that the unit is  firmly  fixed  to avoid  vibration  and  slipping  while  operating.  And  make  sure  that  the structure to which you are attaching the unit is secure and is able to support a weight of 10 times of the unit’s weight. Also always use a safety cable that can hold 12 times of the weight of the unit when installing the fixture.

The equipment must be fixed by professionals. And it must be fixed at a place where is out of the touch of people.

2. Technical Specifications

Light Sources: 12*18W 6in1 RGBWA+UV LED

Beam Angle: 25°

Power Voltage: AC 100-240V, 50/60Hz

Net Weight: 5Kgs

Control

DMX Channel: 6/8/10 Channel

Control Modes: DMX512, wireless DMX and Wifi control

Construction

Display: LED Display

Data In/Out socket: 3-pin XLR sockets

Power Socket: Powercon in/out

Protection Rating: IP65

Features

* Auto power

* IP65 Powercon in/Out ,IP65 DMX in/out

* Option for change LED Power (Eco, Medium, High power)

* Battery Charging Information and Wireless Channel change in Display

* Emergency Mode (enable / disable)

* DMX hold （on/off）in display

* Timer on/off

3. How To Control The Unit

The DMX512 is widely used in intelligent lighting control,with a DMX 512 controller.connect several lights together,dmx in and dmx out,

3 pin XLR connectors: Pin 1: GND, Pin 2: Negative signal (-), Pin 3: Positive signal (+)

Display:

MENU	To select the programming functions

DOWN	To go backward in the selected functions

UP	To go forward in the selected functions

ENTER	To confirm the selected functions

Set DMX Address Code

1	Press "Menu" to "Set DMX Address",and press"ENTER" keys to enter into

2	Show "Set DMX Address DMX Address:001",Press the "UP and DOWN"

keys to amended

- Press "ENTER"keys to save and Exit,Press the"MENU"Keys does not save and Exit.

- Turn on operation: when in charging or turn off, press "MENU" for more than 4s,then screen will display clients' logo and model of products,

- there are "MENU","UP"."DOWN" for your selection to choose the function,press" ENTER" to confirmed and enter the next menu,press "MENU" could back to the last menu.

- When turn on , press" MENU" for more than 4s,the screen will display " Power Off",and will close LED output by then.

- If charger connect with electricity,the screen will display charging icon and percentage of battery power. Or will be turn off directly,and the battery

- powered will be totally cut off.

- When using Master-Slave model,apart from the first one as Master without setting,the others need to set as Slave.

- When using DMX512 controller,all the lights need to set as DMX model.

MENU

| Menu |  | Function |  |
| --- | --- | --- | --- |
| A001 | A001—A512） | DMX Address 001-512 |  |
| Chnd | 6CH | 6 channels mode |  |
|  | 8CH | 8 channels mode |  |
|  | 10CH | 10 channels mode |  |
| FAIL | BLND | Data is not maintained (black field) |  |
|  | HOLd | Data retention |  |
| CL01 | CL01—CL32 | 32 static color mixing modes, color selection by up and down keys Strobe mode St00-St16 after confirmation, adjust by up and down keys |  |
| AUT0 | AUT0 | Auto mode 0, speed adjustable SP01-SP16 |  |
|  | AUT1 | Auto mode 1, speed adjustable SP01-SP16 |  |
|  | AUT2 | Auto mode 2, speed adjustable SP01-SP16 |  |
|  | AUT3 | Auto mode 3, speed adjustable SP01-SP16 |  |
|  | AUT4 | Auto mode 4, speed adjustable SP01-SP16 |  |
| SOU0 | SOU0 | Sound mode 0 |  |
|  | SOU1 | Sound mode 1 |  |
|  | SOU2 | Sound mode 2 |  |
|  | SOU3 | Sound mode 3 |  |
| WIFI | LED-XXX | WIFI |  |
| NANU | R000-255 | R manual dimming |  |
|  | G000-255 | G manual dimming |  |
|  | B000-255 | B manual dimming |  |
|  | U000-255 | W manual dimming |  |
|  | Y000-255 | A manual dimming |  |
|  | P000-255 | UV manual dimming |  |
|  | S000-255 | Strobe manual dimming |  |
|  | d000-255 | Brightness manual dimming |  |
| TEST |  | Color self-check |  |
| LED | ON | Display ON |  |
|  | OFF | Display OFF |  |
| IRCL | ON | IR remote control ON |  |
|  | OFF | IR remote control OFF |  |
| 2.4G | ON | Wireless 2.4G ON (press the ENTER button for 3 seconds to enter the frequency modulation: 1-7) |  |
|  | OFF | Wireless 2.4G OFF |  |
| POUR | ON | One key switch machine, ON | Pressed MENU button for 3 seconds to turn ON/OFF |  |
|  | OFF | One key switch machine, OFF |  |  |
| VER | V1.0 | Software version |  |

DMX channel´s functions and values (6/10/10 DMX channels):

| CH | 6CH | 8CH | 10CH |
| --- | --- | --- | --- |
| CH1 | 0-255 Red | 0-255 Dimmer | 0-255 Dimmer |
| CH2 | 0-255 Green | 0-10 OFF 11-255 Strobe (Slow-fast) | 0-255 Red |
| CH3 | 0-255 Blue | 0-255 Red | 0-255 Green |
| CH4 | 0-255 White | 0-255 Green | 0-255 Blue |
| CH5 | 0-255 Amber | 0-255 Blue | 0-255 White |
| CH6 | 0-255 UV | 0-255 White | 0-255 Amber |
| CH7 |  | 0-255 Amber | 0-255 UV |
| CH8 |  | 0-255 UV | 0-9 OFF， 10-255 Strobe (Slow-fast) |
| CH9 |  |  | 0-9 OFF 10-50 Static Colors(Ch10 choose color) 51-100 Colors Jump 101-150 Colors Gradual 151-200 Colors Pulse 201-220 Sound mode 1 221-240 Sound mode 2 241-255 Sound mode 3 |
| CH10 |  |  | 0-255 Speed(slow to fast) |

- 4.Remind

*It need to charge a times for each two months,and 50% battery power at least, if for a long time without using.

When turned off the lights and charging, the battery information in display shows 100%, please continue charging more than 1 hours, then get maximum discharge time.