- Key operation and digital tube display instructions

1. Key description

MENU: function key	UP: Key DOWN: key ENTER: Confirm key

2. Digital tube display menu description

| number | content | value | function declaration |  |
| --- | --- | --- | --- | --- |
| 1 | A001 | 001-512 | The DMX512 address setting |  |
| 2 | 9CH | 4/6/9/40/42 | Channel mode setting |  |
| 3 | R255 | 000 - 255 | Red dimming, from dark to bright |  |
| 4 | G255 | 000 - 255 | Green dimming, from dark to bright |  |
| 5 | B255 | 000 - 255 | Blue dimming, from dark to bright |  |
| 6 | W255 | 000 - 255 | White dimming, from dark to bright |  |
| 7 | F000 | 005 - 255 | Total strobe, from slow to fast |  |
| 8 | CL01 | 01 - 08 | red, green, blue, white, yellow, purple, cyan, RGBW |  |
| 9 | EF01 | 01 - 20 | UP to switch the effect | Press ENTER to set the speed SP01-SP15 |
| 10 | SU01 | 01 - 06 | Sound mode | Press ENTER to set the sensitivity SE01-SE15 |

RGBW 10-eyes

- The DMX channel menu

9 Channel (Basic Function)

| channel | value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming, from dark to bright |
| CH 2 | 005-255 | Total strobe, from slow to fast |
| CH 3 | 000-009 | Perform the (CH 6-9 ) RGBW static dimming |
|  | 010-019 | Static color, CH4 Select (red, green, blue, white, yellow, purple, cyan, RGBW) |
|  | 020-029 | Breathing effect, CH5 Select color |
|  | 030-039 | Jump change effect |
|  | 040-049 | Gradient change effect |
|  | 050-059 | Pulse change effect, CH5 Select color |
|  | 060-069 | Autorun effect-1, CH5 Select color(cycle autorun) |
|  | 070-079 | Autorun effect-2, CH5 Select color |
|  | 080-089 | Autorun effect-3, CH5 Select color |
|  | 090-099 | Autorun effect-4, CH5 Select color |
|  | 100-109 | Autorun effect-5, CH5 Select color |
|  | 110-119 | Autorun effect-6, CH5 Select color |
|  | 120-129 | Autorun effect-7, CH5 Select color |
|  | 130-139 | Autorun effect-8, CH5 Select color |
|  | 140-149 | Autorun effect-9, CH5 Select color |
|  | 150-159 | Autorun effect-10, CH5 Select color |
|  | 160-169 | Autorun effect-11, CH5 Select color |
|  | 170-179 | Autorun effect-12, CH5 Select color |
|  | 180-189 | Autorun effect-13, CH5 Select color |
|  | 190-199 | Autorun effect-14, CH5 Select color |
|  | 200-209 | Autorun effect-15, CH5 Select color |
|  | 210-229 | Autorun effect-16, CH5 Select color (8 full color effects) |
|  | 230-233 | Sound mode 1, CH5 Select color |
|  | 234-237 | Sound mode 2, CH5 Select color |
|  | ... | ... |
|  | 246-249 | Sound mode 5, CH5 Select color |
|  | 250-255 | Sound mode 6, CH5 Select color |
| CH 4 | 000-255 | Static color mode: red, green, blue, white, yellow, purple, cyan, RGBW. Autorun effect: adjustment speed, from slow to fast . Sound mode: adjustment sensitivity, from low to high (The highest sensitivity is the sensitivity set by the menu) . |
| CH 5 | 000-031 | Default color |
|  | 032-063 | Red, CH3 Manual color selection |
|  | 064-095 | Green, CH3 Manual color selection |
|  | 096-127 | Blue, CH3 Manual color selection |
|  | 128-159 | Yellow, CH3 Manual color selection |
|  | 160-191 | Purple, CH3 Manual color selection |
|  | 192-223 | Cyan, CH3 Manual color selection |
|  | 224-255 | White, CH3 Manual color selection |
| CH 6 | 000-255 | R red dimming, from dark to bright |
| CH 7 | 000-255 | G Green dimming, from dark to bright |
| CH 8 | 000-255 | B Blue dimming, from dark to bright |
| CH 9 | 000-255 | W White dimming, from dark to bright |

- 4 Channel (whole lamp)

| channel | value | function declaration |
| --- | --- | --- |
| CH 1 | 000-255 | R red dimming, from dark to bright |
| CH 2 | 000-255 | G Green dimming, from dark to bright |
| CH 3 | 000-255 | B Blue dimming, from dark to bright |
| CH 4 | 000-255 | W White dimming, from dark to bright |

- 6 Channel (whole lamp)

| channel | value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming, from dark to bright |
| CH 2 | 005-255 | Total strobe, from slow to fast |
| CH 3 | 000-255 | R red dimming, from dark to bright |
| CH 4 | 000-255 | G Green dimming, from dark to bright |
| CH 5 | 000-255 | B Blue dimming, from dark to bright |
| CH 6 | 000-255 | W White dimming, from dark to bright |

40 Channel (point control)

| channel | value | function declaration |
| --- | --- | --- |
| CH 1 | 000-255 | R1 red dimming, from dark to bright |
| CH 2 | 000-255 | G1 green dimming, from dark to bright |
| CH 3 | 000-255 | B1 blue dimming, from dark to bright |
| CH 4 | 000-255 | W1 white dimming, from dark to bright |
| ... | ... | ... |
| ... | ... | ... |
| CH 37 | 000-255 | R10 red dimming, from dark to bright |
| CH 38 | 000-255 | G10 green dimming, from dark to bright |
| CH 39 | 000-255 | B10 blue dimming, from dark to bright |
| CH 40 | 000-255 | W10 white dimming, from dark to bright |

42 Channel (point control)

| channel | value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming, from dark to bright |
| CH 2 | 005-255 | Total strobe, from slow to fast |
| CH 3 | 000-255 | R1 red dimming, from dark to bright |
| CH 4 | 000-255 | G1 green dimming, from dark to bright |
| CH 5 | 000-255 | B1 blue dimming, from dark to bright |
| CH 6 | 000-255 | W1 white dimming, from dark to bright |
| ... | ... | ... |
| ... | ... | ... |
| CH 39 | 000-255 | R10 red dimming, from dark to bright |
| CH 40 | 000-255 | G10 green dimming, from dark to bright |
| CH 41 | 000-255 | B10 blue dimming, from dark to bright |
| CH 42 | 000-255 | W10 white dimming, from dark to bright |