AMIXLED LED BAR 24X4 BATTERY IP65

This manual contains important information. Please read before operating fixture.

This manual contains important information. Please read before operating fixture.

- General safety tips

WARNING!!! To reduce the risk of fire, electric shock, or injury to persons, follow these important safety instructions:

WARNING!!! To reduce the risk of fire, electric shock, or injury to persons, follow these important safety instructions:

  - non-professionals, not to disassemble the light arbitrarily and lighting accessories inside.

  - rated voltage range: AC110V-120V/60Hz or AC200V-240V / 50HZ (to be selected).

  - not beam irradiate the flammable materials, flammable materials and the lamp should be maintained at least 1M over the distance.

  - lamps used in ambient temperature: -20 ℃ -40 ℃ , the highest temperature of the surface of the lamp: 60 ℃.

  - to keep away from the liquid substance and humid environment.

  - before using the lights must ensure the good grounding, cannot install charging and remove any parts.

  - the installation of lamps, must be fixed screw fasteners with additional security cable and regular inspection.

  - lamps consecutive working hours is not recommended more than 10 hours.

  - should stop using the lights in time when happens unusual conditions during the process.

  - lamp parts of the rotation, external accessories and paste parts should be regular inspected, such as the emergence of loose and rock, should be reinforced in a timely manner to prevent the accident.

- Technical Specifications

Power Supply: AC 100-240V/50-60Hz Power consumption:100W

Light source: 24pcs*4w LED RGBW 4in1.

LED feature: Create dazzling effects with 2 /8 zones of individual control. Beam Angle: 25 degrees

Dimmer: 0-100% dimmer Strobe rate:0-24HZ

Working Environment: -20℃-40℃

IP rate: IP65

DMX Channels:6/8/14/50ch

Control Mode: DMX512, Master-slave, Sound, Auto, remote, wireless Control panel: LED display

Power connection: Powercon in out DMX control: 3pin XLR DMX in out Battery type: Lithium Ion

Battery Power: 12V, 27Ah

Battery lifespan: 500 times charging/discharging circles. Battery Charge Time: 6hours (full power)

Battery Life: (all on) 4.5hours, (single Color) 18 hours

- On-board Menu layout

Operate menu :

| Addr | A001-A512 | DMX Address selection |
| --- | --- | --- |
| Chnd | 6/8/14/50ch | Channel selection |
| Slnd | MAst/d512/SlAU | Master slave selection |
| Soun | So 0-16 | Sound control effect selection |
|  | Se 99-0 | Sound control sensitivity adjustment |
| Auto | Au 0- Au16 | Auto effect selection |
|  | SP 0-SP 9 | Auto effect speed adjustment |
| Colo | Co 1-Co 12 | Color selection |
| Manu | Red 0-255 | Red dimmer |
|  | Gree 0-255 | Green dimmer |
|  | Blue 0-255 | Blue dimmer |
|  | Whit 0-255 | White dimmer |
| disp | ds!p | Digital display flip setting |
| 2-4G | On/off | Wireleess on/off |
| ver | V10 | Current software vertion |

- DMX Channel Modes

| 6 Channel |  |  |
| --- | --- | --- |
| CH | Value | Function |
| 1 | 0-255 | Red 0-100% |
| 2 | 0-255 | Green 0-100% |
| 3 | 0-255 | Blue 0-100% |
| 4 | 0-255 | White 0-100% |
| 5 | 0-255 | Color Chase, See Color Chase Table |
| 6 | 0-255 | Chase Speed Slow To Fast |

| 8 Channel |  |  |
| --- | --- | --- |
| CH | Value | Function |
| 1 | 0-255 | All dimming |
| 2 | 0-255 | Strobe |
| 3 | 0-255 | Red 0-100% |
| 4 | 0-255 | Green 0-100% |
| 5 | 0-255 | Blue 0-100% |
| 6 | 0-255 | White 0-100% |
| 7 | 0-255 | Color Chase, See Color Chase Table |
| 8 | 0-255 | Chase Speed Slow To Fast |

| 14 Channel |  |  |
| --- | --- | --- |
| CH | Value | Function |
| 1 | 0-255 | All dimming 1 |
| 2 | 0-255 | Strobe |
| 3 | 0-255 | Red 1 0-100% |
| 4 | 0-255 | Green 1 0-100% |
| 5 | 0-255 | Blue 1 0-100% |
| 6 | 0-255 | White1 0-100% |
| 7 | 0-255 | All dimming 2 |
| 8 | 0-255 | Strobe |
| 9 | 0-255 | Red 2 0-100% |
| 10 | 0-255 | Green 2 0-100% |
| 11 | 0-255 | Blue 2 0-100% |
| 12 | 0-255 | White 2 0-100% |
| 13 | 0-255 | Color Chase, See Color Chase Table |
| 14 | 0-255 | Chase Speed Slow To Fast |

| 50 Channel |  |  |
| --- | --- | --- |
| CH | Value | Function |
| 1 | 0-255 | All dimming 1 |

| 2 | 0-255 | Strobe1 |
| --- | --- | --- |
| 3 | 0-255 | Red 1 0-100% |
| 4 | 0-255 | Green 1 0-100% |
| 5 | 0-255 | Blue 1 0-100% |
| 6 | 0-255 | White 1 0-100% |
| 7 | 0-255 | All dimming 2 |
| 8 | 0-255 | Strobe2 |
| 9 | 0-255 | Red 2 0-100% |
| 10 | 0-255 | Green 2 0-100% |
| 11 | 0-255 | Blue 2 0-100% |
| 12 | 0-255 | White 2 0-100% |
| 13 | 0-255 | All dimming 3 |
| 14 | 0-255 | Strobe3 |
| 15 | 0-255 | Red 3 0-100% |
| 16 | 0-255 | Green 3 0-100% |
| 17 | 0-255 | Blue 3 0-100% |
| 18 | 0-255 | White 3 0-100% |
| 19 | 0-255 | All dimming 4 |
| 20 | 0-255 | Strobe4 |
| 21 | 0-255 | Red 4 0-100% |
| 22 | 0-255 | Green 4 0-100% |
| 23 | 0-255 | Blue 4 0-100% |
| 24 | 0-255 | White 4 0-100% |
| 25 | 0-255 | All dimming 5 |
| 26 | 0-255 | Strobe5 |
| 27 | 0-255 | Red 5 0-100% |
| 28 | 0-255 | Green 5 0-100% |
| 29 | 0-255 | Blue 5 0-100% |
| 30 | 0-255 | White 5 0-100% |
| 31 | 0-255 | All dimming 6 |
| 32 | 0-255 | Strobe6 |

| 33 | 0-255 | Red 6 0-100% |
| --- | --- | --- |
| 34 | 0-255 | Green 6 0-100% |
| 35 | 0-255 | Blue 6 0-100% |
| 36 | 0-255 | White 6 0-100% |
| 37 | 0-255 | All dimming 7 |
| 38 | 0-255 | Strobe7 |
| 39 | 0-255 | Red 7 0-100% |
| 40 | 0-255 | Green 7 0-100% |
| 41 | 0-255 | Blue 7 0-100% |
| 42 | 0-255 | White 7 0-100% |
| 43 | 0-255 | All dimming 8 |
| 44 | 0-255 | Strobe8 |
| 45 | 0-255 | Red 8 0-100% |
| 46 | 0-255 | Green 8 0-100% |
| 47 | 0-255 | Blue 8 0-100% |
| 48 | 0-255 | White 8 0-100% |
| 49 | 0-255 | Color Chase, See Color Chase Table |
| 50 | 0-255 | Chase Speed Slow To Fast |

Color chase table

| Color chase | Value | Function |
| --- | --- | --- |
|  | 000-007 | No function |
|  | 008-022 | Color chase 01 |
|  | 023-037 | Color chase 02 |
|  | 038-052 | Color chase 03 |
|  | 053-067 | Color chase 04 |
|  | 068-082 | Color chase 05 |
|  | 083-097 | Color chase 06 |
|  | 098-112 | Color chase 07 |
|  | 113-127 | Color chase 08 |

|  | 128-142 | Color chase 09 |
| --- | --- | --- |
|  | 143-157 | Color chase 10 |
|  | 157-172 | Color chase 11 |
|  | 173-187 | Color chase 12 |
|  | 188-202 | Color chase 13 |
|  | 203-217 | Color chase 14 |
|  | 218-232 | Color chase 15 |
|  | 233-247 | Color chase 16 |
|  | 248-255 | Sound effect 1-16 |

- Installing the fixture

Please fasten the fixture clamp before hanging. The clamp can be fastened by the little hole in the middle of the bracket, then tie with a safety belt. The loading capacity of the safety belt should be 10 times than the weight of the fixture itself.

- DMX protocol connection

Please connect XLR-XLR control wire with every light from the output of DMX.

XLR connection:

ATTENTION: Please do remember to connect one circle circuit plug on the output signal of last equipment. And this circle circuit plug was connected a 120ohm resistance between terminals 2 and 3 of the CANON plug, connect

this circle circuit plug can avoid the signal flash phenomena of signal DMX512 during the transmission.

- Maintenance/Troubleshooting

| Problems | Check list | Trouble Shooting |
| --- | --- | --- |
| The fixture does not work. | - If your power as same as the input of the machine. - If the fuse was broken. | - Make sure the voltage. - Replace the same fuse. |
| LED dimmed. | If your power us same as the input of the machine | Make sure the voltage input. |
| No DMX input. | - If the polarity is opposite in the DMX controller. - If it is in DMX mode. - If there are 2 Master in a loop. | - Correct the DMX polarity. - Change to the DMX mode. - Set one piece into Slave mode. |
| Bar Light works abnormality | - Check if the serial is correct on the Par Light display | - Set the right serial No on the single Par Light |



Warranty Card

Name

Model

Name

Model

Production	Purchase

Production	Purchase

End User Company Name:

Add:

End User Company Name:

Add:

Sale Store

Link man: Tel:

Stamp

Sale Store

Link man: Tel:

Stamp

Description: These must be truthfully fill, seal by the end-user and point-of-sale, or else the warranty will not work (warranty period and terms, see the following)

warranty period：one year since purchase warranty terms：1、If fail when using the product in

normal state, you can made under the provisions of the warranty, show the warranty card and purchase bills(copy), and enjoy free maintenance service in a specific service center or the manufacturer.

- The following will be the implementation of paid service:

  - Without the effective warranty card

  - There’s blank, alter and no point-of-sale name

  - Fail caused by power supply

  - Fail caused by transportation or unloading

  - Fail caused by not operate by manual

  - Fail caused by disassemble without unauthorized

  - Fail caused by using the unauthorized control system

Note:

Based on our company continue to improve the product, the statement containing data may have changed, and without any notice. Company retains its right to change specifications related when product is improved. Thanks for your understanding.