
200W LED Beam Moving Head with Halo
User Manual

[image: image1.png]



	This manual contains important information. Please read before operating fixture.                                            


                                                                                                             1  SECURITY TIPS
	WARNING!!! To reduce the risk of fire, electric shock, or injury to persons, follow these important safety instructions:


non-professionals, not to disassemble the light arbitrarily and lighting accessories inside. 

rated voltage range: AC110V-120V/60Hz or AC200V-240V / 50HZ (to be selected). 

not beam irradiate the flammable materials, flammable materials and the lamp should be maintained at least 1M over the distance. 

lamps used in ambient temperature: -15 ℃ -40 ℃, the highest temperature of the surface of the lamp: 60 ℃. 

to keep away from the liquid substance and humid environment. 

before using the lights must ensure the good grounding, can not install charging and remove any parts. 

the installation of lamps, must be fixed screw fasteners with additional security cable and regular inspection. 

lamps consecutive working hours is not recommended more than 10 hours. 

should stop using the lights in time when happens unusual conditions during the process 

lamp parts of the rotation, external accessories and paste parts should be regular inspected, such as the emergence of loose and rock, should be reinforced in a timely manner to prevent the accident. 

lamps adopt strong winds to refrigerate, it is easy to accumulate the dust, so you have to clean it weekly especially the cooling air opening, otherwise the dust will plug leading to poor light
2  TECHNOLOGY REFERENCE

Voltage: AC110-240V,50/60Hz

Total power: 200W

Color wheel: 13 colors+white

Gobo wheel：11 patterns+White 

Operating temperature: -10°～50°

Color temperature: 8000K

Control mode: DMX512, master-slave mode, Sound
Prism: Single prism

Ra: 70

Protection level: IP20

Size：28*18*46CM

Net weight：7.7KG
3  Board Instruction
Operate Menu 

	menu operation settings

	primary menu
	Submenu 1
	Function setting
	Brief introduction of function

	DMX address
	
	1-512
	

	system setup
	Display backward
	normal
	.

	
	
	turn
	

	
	Language settings
	Chinese
	

	
	
	English
	

	
	Channel mode
	20CH
	

	
	operational mode
	DMX
	

	
	
	sound-controlled
	

	
	
	automatic
	

	
	Electronic lighting
	close
	

	
	
	open
	

	
	Bulb default
	On _ off
	

	
	Light bulb control
	illumine
	

	
	shortest route
	On _ off
	

	
	Horizontal reversal
	close
	

	
	
	open
	

	
	Vertical inversion
	close
	

	
	
	open
	

	
	XY speed
	low speed
	

	
	
	intermediate speed
	

	
	
	fast
	

	
	Optocoupler detection
	close
	

	
	
	open
	

	
	Return to the superior
	▲
	

	information inquiry
	Total time
	0-9999
	

	
	Bulb time
	0-9999
	

	
	binding hours
	0-9999
	

	
	display software
	Vx001
	

	
	XY software
	Vx001
	

	
	Small motor software
	Vx001
	

	
	Return to the superior
	▲
	

	Channel control
	Color wheel rotation
	0-255
	

	
	stroboscopic
	0-255
	

	
	Dimming "
	0-255
	

	
	Fixed pattern
	0-255
	

	
	prism
	0-255
	

	
	Prism rotation
	0-255
	

	
	Fogging
	0-255
	

	
	Focus adjustment
	0-255
	

	
	Horizontal rotation
	0-255
	

	
	Horizontal fine adjustment
	0-255
	

	
	Vertical rotation
	0-255
	

	
	Vertical fine tuning
	0-255
	

	
	reset
	0-255
	

	
	light a lamp
	0-255
	

	
	XY speed
	0-255
	

	
	Color wheel speed
	0-255
	

	
	Focusing speed
	0-255
	

	
	Pattern speed
	0-255
	

	
	Return to the superior
	0-255
	

	functional mode
	XY communication
	normal
	current state

	
	
	mistake
	

	
	Small motor communication
	normal
	

	
	
	mistake
	

	
	Focusing hall
	normal
	

	
	
	mistake
	

	
	Color wheel hall
	normal
	

	
	
	mistake
	

	
	Pattern hall
	normal
	

	
	
	mistake
	

	
	X-optical coupler
	normal
	

	
	
	mistake
	

	
	Y optocoupler
	normal
	

	
	
	mistake
	

	
	Fan speed
	6500
	

	
	Return to the superior
	▲
	

	appurtenance
	Check code 1
	0-9999
	

	
	Total time
	clear
	

	
	
	carry out
	

	
	Lamp time
	clear
	

	
	
	carry out
	

	
	Restore default
	default
	

	
	
	recover
	

	
	
	preserve
	

	
	Horizontal rotation
	0-255
	Zero initial position

	
	Vertical rotation
	0-255
	

	
	Focus adjustment
	0-255
	

	
	Prism 1
	0-255
	

	
	Prism 2
	0-255
	

	
	Prism rotation 1
	0-255
	

	
	Prism rotation 2
	0-255
	

	
	Fogging
	0-255
	

	
	Color combination
	0-255
	

	
	Fixed pattern
	0-255
	

	
	colour disc
	0-255
	

	
	Frequency flash
	0-255
	

	
	Stroboscopic midpoint
	0-255
	

	
	Ridge-1 midpoint
	0-255
	

	
	Mid-point of edge two
	0-255
	

	
	Atomization midpoint
	0-255
	

	
	Colorful midpoint
	0-255
	

	
	Return to the superior
	▲
	

	Advanced settings
	Check code 2
	0-9999
	

	
	Signal encryption
	open
	

	
	
	close
	

	
	DMX speed
	0-255
	

	
	DMX data packet
	0-255
	

	
	DMX type
	0-055
	

	
	DMX isolation
	0-65535
	The smaller the value, the more stable the signal is.

	
	Driving plate temperature
	< 110 degrees >
	If it exceeds this range, red will be displayed.

	
	Drive board 3.3V
	3.0V - 3.6V
	If it exceeds this range, red will be displayed.

	
	Drive board 5.0V
	4.5V - 5.5V
	If it exceeds this range, red will be displayed.

	
	Drive board 12V
	10.0V - 14.0V
	If it exceeds this range, red will be displayed.

	
	Drive plate 36V
	20.0V - 42.0V
	If it exceeds this range, red will be displayed.

	
	X state
	be motionless
	

	
	
	turn
	

	
	X hall
	normal
	

	
	
	mistake
	Show red

	
	X-ray optical coupler phase
	0-3
	

	
	X optocoupler a
	40-160
	The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm.

	
	X optocoupler b
	40-160
	The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm.

	
	X optocoupler c
	40-160
	The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm.

	
	X optocoupler d
	40-160
	The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm.

	
	X current angle
	000-540
	

	
	X drive state
	normal
	

	
	
	mistake
	Show red

	
	Y state
	be motionless
	

	
	
	turn
	

	
	Y optocoupler
	normal
	

	
	
	mistake
	Show red

	
	Y hall
	normal
	

	
	
	mistake
	Show red

	
	Y-coupler phase
	0-3
	

	
	Y optocoupler a
	40-160
	If it exceeds this range, red will be displayed.

	
	Y optocoupler b
	40-160
	If it exceeds this range, red will be displayed.

	
	Y optocoupler c
	40-160
	If it exceeds this range, red will be displayed.

	
	Y optocoupler d
	40-160
	If it exceeds this range, red will be displayed.

	
	Y current angle
	000-250
	

	
	Y drive state
	normal
	

	
	
	mistake
	Show red

	
	Return to the superior
	▲
	

	Channel level
	Color wheel rotation
	0-255
	Current DMX channel value

	
	stroboscopic
	0-255
	

	
	aiming
	0-255
	

	
	Fixed pattern
	0-255
	

	
	prism
	0-255
	

	
	Prism rotation
	0-255
	

	
	Fogging
	0-255
	

	
	Focus adjustment
	0-255
	

	
	Horizontal rotation
	0-255
	

	
	Horizontal fine adjustment
	0-255
	

	
	Vertical rotation
	0-255
	

	
	Vertical fine tuning
	0-255
	

	
	reset
	0-255
	

	
	light a lamp
	0-288
	

	
	XY speed
	0-255
	

	
	Color wheel speed
	0-255
	

	
	Focusing speed
	0-255
	

	
	Pattern speed
	0-255
	

	
	Return to the superior
	
	

	reset
	Reset all
	cancel
	manual reset

	
	
	carry out
	

	
	XY reset
	cancel
	

	
	
	carry out
	

	
	Effect reset "
	cancel
	

	
	
	carry out
	

	
	Return to the superior
	
	

	Exit menu
	
	
	


4  DMX CHANNEL SHEET 

20 Channels 
	Passage 
	name
	numerical value
	describe

	CH1
	CH1
	colour
	0-4
	white light

	
	
	
	5-9
	White light+color 1

	
	
	
	10-14
	Color 1

	
	
	
	15-19
	Color 1+ Color 2

	
	
	
	20-24
	Color 2

	
	
	
	25-29
	Color 2+ color 3

	
	
	
	30-34
	Color 3

	
	
	
	35-39
	Color 3+ color 4

	
	
	
	40-44
	Color 4

	
	
	
	45-49
	Color 4+ color 5

	
	
	
	50-54
	Color 5

	
	
	
	55-59
	Color 5+ color 6

	
	
	
	60-64
	Color 6

	
	
	
	65-69
	Color 6+ color 7

	
	
	
	70-74
	Color 7

	
	
	
	75-79
	Color 7+ color 8

	
	
	
	80-84
	Color 8

	
	
	
	85-89
	Color 8+ color 9

	
	
	
	90-94
	Color 9

	
	
	
	95-99
	Color 9+color 10

	
	
	
	100-104
	Color 10

	
	
	
	105-109
	Color 10+color 11

	
	
	
	110-114
	Color 11

	
	
	
	115-119
	Color 11+color 12

	
	
	
	120-124
	Color 12

	
	
	
	125-129
	Color 12+color 13

	
	
	
	130-134
	Color 13

	
	
	
	135-140
	Color 13+white light

	
	
	
	141-150
	white light

	
	
	
	151-205
	From fast to slow forward flow

	
	
	
	206-255
	Reverse flow from slow to fast

	CH2
	CH2
	stroboscopic
	0-3
	Shutter closed

	
	
	
	4-103
	Synchronous stroboscopic

	
	
	
	104-107
	Shutter open

	
	
	
	108-207
	Flat frequency flash

	
	
	
	208-212
	Shutter open

	
	
	
	213-225
	Low speed random strobe

	
	
	
	226-238
	Medium speed random strobe

	
	
	
	239-251
	High speed random strobe

	
	
	
	252-255
	Shutter open

	CH3
	CH3
	aiming
	0-255
	0-100% dimming

	CH4
	CH4
	pattern
	0-7
	Baiguangkong

	
	
	
	8-14
	Pattern 1

	
	
	
	15-21
	Pattern 2

	
	
	
	22-28
	Pattern 3

	
	
	
	29-35
	Pattern 4

	
	
	
	36-42
	Pattern 5

	
	
	
	43-49
	Pattern 6

	
	
	
	50-57
	Pattern 7

	
	
	
	58-64
	Pattern 8

	
	
	
	65-71
	Pattern 9

	
	
	
	72-78
	Pattern 10

	
	
	
	79-85
	Pattern 11

	
	
	
	86-90
	Pattern 12

	
	
	
	91-99
	white light

	
	
	
	100-209
	Pattern jitter from slow to fast.

	
	
	
	210-230
	The pattern is turning from slow to fast.

	
	
	
	231-234
	Stop turning

	
	
	
	235-255
	Pattern inversion from slow to fast

	CH5
	CH5
	prism
	0-127
	Prismatic

	
	
	
	128-255
	Prism cut in

	CH6

	CH6
	Prism rotation
	0-127
	0-400 degrees

	
	
	
	128-190
	Reverse flow from fast to slow

	
	
	
	191-192
	stop

	
	
	
	193-255
	From slow to fast forward flow

	CH7   Rainbow

	0-127
	without

	
	128-255
	Insert colorful

	CH8   Frost

	0-127
	without

	
	128-255
	Insertion Frost

	CH9
	CH9
	focus
	0-255
	From far to near

	CH 10
	CH10
	X axis
	0-255
	0-540 degrees

	CH 11
	CH11
	X-axis fine tuning
	0-255
	X-axis 16bit fine tuning channel

	CH 12
	CH12
	Y axis
	0-255
	0-240 degrees

	CH 13
	CH13
	Y axis fine tuning
	0-255
	Y-axis 16bit fine tuning channel

	CH 14

	CH14 lamp strip
	
	0-255
	The effect goes from slow to fast.

	CH 15 reset

	CH15
	Horizontal vertical velocity
	0-255
	Fast to slow

	CH 16 is empty
	CH16
	reset
	0-127
	without

	
	
	
	128-255
	Complete machine reset


5  DIFFICULTY RESOLUTION
The following is a list of problems and resolution, and other malfunction preparing work should be professionally qualified staff to deal with. 

1. Bulb not working 
Check whether approaching the service life of light bulbs, then change it
Check whether bulbs circuitry leak, loss or connect poorly
2. Beam appears bleak
The bulbs maybe approach the service life, then change it, or the optical components are unclean.
Bulbs have accumulation of dust, so the bulbs and other parts inside the lamp should be cleaned and maintained in regular period.
3. Computer lamp work intermittently
Check whether the fan process naturally and become dirty
Check inner temperature-response control plug is off
Check whether the circuitry is good
4. The computer lamp not controlled by controller during lighting
Check numeral starting address and the connection of communication control circuit
The corresponding channel button of controller is open or not 
Check whether the fuse on power supply input jack is broken off
6  AFTER SERVICE
Our company offer technical consultation to customers for ever.
If the machine is wrong and need repairmen, please truthfully fill in the related content and feedback us the problems timely, as well as we have to improve the product.
In addition to the lamp, also matching accessories are as follows：
accessories：Power line  1 piece     Optional parts：(customers）
(
                 Warranty Card
        Name     Model       Production  Purchase

	End User
Company Name:
Add:


	Sale Store
Link man: 
Tel:             Stamp



Description: These must be truthfully fill, seal by the end-user and point-of-sale, or else the warranty will not work (warranty period and terms, see the following)
warranty period：one year since purchase warranty terms：1、If fail when using the product in normal state, you can made under the provisions of the warranty, show the warranty card and purchase bills(copy), and enjoy free maintenance service in a specific service center or the manufacturer. 
2. The following will be the implementation of paid service: 
    (1)Without the effective warranty card
    (2)There’s blank, alter and no point-of-sale name
    (3)Fail caused by unresistance  
    (4)Fail caused by transportation or unloading 
    (5)Fail caused by not operate by manual
    (6)Fail caused by disassemble without unauthorized
      (7)Fail caused by using the unauthorized control system  
Note: 

Based on our company continue to improve the product, the statement containing data may have changed, and will not notice the change any more. Company retains its right to change specifications related when product is improved. Thanks for your understanding.

 11

