
   300W LED Beam Moving Head
[image: image1.png][daZel

e e




 [image: image4.png]


 

User Manual

Thank you for choosing our LED 300W moving head light. For your own safety, please read this manual before installing the device. This manual covers the important information on installation and applications. Please install and operate the fixture with following instructions. Meanwhile, please keep this manual well for future needs.

1. Safety Tips

    To ensure your safety and correct use of lamps. Please read the following safety tips carefully before use to avoid unnecessary failures and injuries.

1. Non-professionals, do not disassemble the lamps and their accessories without authorization.

2. Rated voltage range: AC110V—120V/250Hz or AC200V—240V /50HZ (must be selected).

3. Do not irradiate the light beam on flammable objects, and keep a distance of at least 1M between the lamp and flammable objects.

4. The ambient temperature of the lamp: -15℃-40℃, the highest temperature of the lamp surface is 40℃.

5. Keep the lamps away from liquid substances and humid environments.

6. Before using the lamp, make sure that the lamp is well grounded, and do not install or remove any parts of the lamp with power on.

7. When installing the lamps, the fixing screws must be fastened, with safety cables, and regular inspections.

8. It is recommended that the continuous working time of the lamp should not exceed 20 hours.

9. During use, if the lamp is abnormal, stop using the lamp in time.

2.Specification

Power consumption 

● Input Voltage: AC90-260V 50/60Hz 

● Power:500W

Lamp

● Main light use 300W WHITE LED,8000K

● 16 RGBW Cree LEDs for HALO Effect

● 20000 hours lifelong time

Optical 

●  Beam angle:3°

●  Liner dim: 0-100%

●  Strobe: 0-20s 

Color wheel

● 1 color wheel: 7 wheels+Blank

Gobo Wheel

● 1 rotating gobo wheel：7 changebale gobo wheel+1 blank

● 1fixed gobo wheel：8 fixed gobo wheel+1 blank

Prism

● A 16 prism and a honeycomb prism, can be superimposed, variable speed rotation

Frost

● Independent fog lens, soft light

Focus

● Electronic focusing: ultra-smoothly adjust the focus, adjust the sharpness of the pattern
Scan

● Driven by a three-phase motor, fast, quiet, with automatic error correction and reset
● Pan：540° 

● Tilt：270° 
Cooling system

● Use powerful silent fan

●Temperature protection function: when the device temperature reaches 40, the fan starts to rotate, when the device reaches 70°

Control

● International standard DMX512 signal control connection

● Control channel mode 25/18CH

● Product size: 330(L)*230(W)*560(H)mm,

● Product weight: net weight: 15.7kg, gross weight: 20kg
3.  Photometric data sheet

[image: image2.jpg](wo) 8715 0909

27000

Lux 180000 110000 80000 52000
——y5 | § |
4m 5m

45cm

Distance (m) 2m 3m

m



4.Menu Display instruction

[image: image3.jpg]LCD Display

Tndicator
MEUN bown
EnTER v




Menu                                                                    


Address setting      [set dmx address]     

                 [DMX mode]

Run       [Auto]       Auto1- Auto 3

                 [Sound Active]

                 [Manual]      Manual mode test      Function and operation

Pan setting
                                     Tilt setting
                                     Display setting
Setting       [ advanced ]            Menu setting
                                      Sensitivity setting (01-100)

                                      Motor Speed
                                      Reset
                                      Factory Setting
                            Dsiplay version 1

System                    DMX address
                            Temperature                   


DMX Chnnel

25/18 channels mode
	25CH
	18CH
	Function
	DMX Value
	   Movement details

	CH1
	CH1
	Pan
	0-255
	Pan 0-540° 

	CH2
	CH2
	Pan fine
	0-255
	Pan Fine

	CH3
	CH3
	Tilt
	0-255
	0-270°

	CH4
	CH4
	Tilt fine
	0-255
	Tilt  Fine

	CH5
	CH5
	Pan Tilt Speed
	0-255
	From fast to slow

	CH6
	CH6
	Dimmer
	0-255
	From dark to bright

	CH7
	CH7
	Strobe
	0-7
	Turn on

	
	
	
	8-250
	Strobe from slow to fast

	CH8
	CH8
	Color
	0-15
	White

	
	
	
	16-31
	Color 1

	
	
	
	32-47
	Color 2

	
	
	
	48-63
	Color 3

	
	
	
	64-79
	Color 4

	
	
	
	80-95
	Color 5

	
	
	
	96-111
	Color 6

	
	
	
	112-127
	Color 7

	
	
	
	128-191
	CW from slow to fast

	
	
	
	192-255
	CCW from slow to fast

	CH9
	CH9
	Color effect
	0-10
	no

	
	
	
	11-209
	Color fine

	
	
	
	210-255
	Color dithering from slow to fast

	CH10
	CH10
	Fixed Gobo wheel
	0-6
	white

	
	
	
	7-13
	Gobo 1

	
	
	
	14-20
	Gobo2

	
	
	
	21-27
	Gobo 3

	
	
	
	28-34
	Gobo 4

	
	
	
	35-41
	Gobo 5

	
	
	
	42-48
	Gobo 6

	
	
	
	49-55
	Gobo 7

	
	
	
	56-63
	Gobo 8

	
	
	
	64-70
	Gobo8 Dithering

	
	
	
	71-77
	Gobo 7 Dithering

	
	
	
	78-84
	Gobo 6 Dithering

	
	
	
	85-91
	Gobo5 Dithering

	
	
	
	92-98
	Gobo4 Dithering

	
	
	
	99-105
	Gobo3 Dithering

	
	
	
	106-112
	Gobo 2 Dithering

	
	
	
	113-119
	Gobo 1 Dithering

	
	
	
	120-127
	white

	
	
	
	128-191
	Clockwise,from slow to fast

	
	
	
	192-255
	Counterclockwise,from slow to fast

	CH11
	CH11
	Rotation wheel
	0-15
	white

	
	
	
	16-31
	Gobo 1

	
	
	
	32-47
	Gobo2

	
	
	
	48-63
	Gobo 3

	
	
	
	64-79
	Gobo 4

	
	
	
	80-95
	Gobo 5

	
	
	
	96-111
	Gobo 6

	
	
	
	112-127
	Gobo 7

	
	
	
	128-191
	Clockwise,from slow to fast

	
	
	
	192-255
	Counterclockwise,from slow to fast

	CH12
	CH12
	Gobo rotation
	0-15
	Gobo Rotation turn on

	
	
	
	16-95
	Gobo rotate 0-250

	
	
	
	96-135
	Gobo rotate 0-90

	
	
	
	136-155
	Gobo rotate 0-250

	
	
	
	156-175
	Gobo rotate 0-360

	
	
	
	176-215
	Gobo rotate CW from slow to fast

	
	
	
	216-255
	Gobo rotate CCW from slow to fast

	CH13
	CH13
	Focus
	0-255
	Gobo hazy to clear

	CH14
	CH14
	Prism 1
	0-63
	No react

	
	
	
	64-255
	Prism in 

	CH15
	CH15
	Prism 2
	0-63
	No react

	
	
	
	64-255
	Prism in 

	CH16
	CH16
	Prism rotate
	0-15
	Prism in 

	
	
	
	16-95
	Prism rotate 0-250

	
	
	
	96-135
	Prism rotate 0-90

	
	
	
	136-155
	Prism rotate 0-250

	
	
	
	156-175
	Prism rotate 0-360

	
	
	
	176-215
	Prism self-rotating CW from slow to fast

	
	
	
	216-255
	Prism self-rotating CCW rotating from slow to fast

	CH17
	 CH17
	Frost
	0-4
	No function

	
	
	
	5-127
	Frost from shallow to deep

	
	
	
	128-191
	Frost shake

	
	
	
	192-255
	Frost effect

	CH18
	
	LED Dimmer
	0-255
	From dark to bright

	CH19
	
	LED Strobe
	0-24
	Turn on 

	
	
	
	25-64
	from fast to slow 

	
	
	
	65-69
	Turn on 

	
	
	
	70-84
	Fast bright and faded from fast to slow

	
	
	
	85-89
	Turn on 

	
	
	
	90-104
	Fast bright and faded from fast to slow

	
	
	
	105-109
	Turn on 

	
	
	
	110-124
	Asynchronous strobe from fast to slow

	
	
	
	125-129
	Turn on 

	
	
	
	130-144
	Asynchronous fast bright and faded from fast to slow

	
	
	
	145-149
	Turn on 

	
	
	
	150-164
	Fast bright and faded from fast to slow

	
	
	
	165-169
	Turn on 

	
	
	
	170-184
	Pulse strobe

	
	
	
	185-189
	Turn on 

	
	
	
	190-204
	Asynchronous Pulse

	
	
	
	205-209
	Turn on 

	
	
	
	210-224
	Fade away

	
	
	
	225-229
	Turn on 

	
	
	
	230-244
	Pulse strobe

	
	
	
	245-255
	Turn on 

	CH20
	
	Head LED

    R  
	0-255
	Dimming from dark to bright

	CH21
	
	Head LED

    G  
	0-255
	Dimming from dark to bright

	CH22
	
	Head LED

    B  
	0-255
	Dimming from dark to bright

	CH23
	
	Head LED

    W  
	0-255
	Dimming from dark to bright

	CH24
	
	LED color changing faded
	0-10
	No function 

	
	
	
	11-20
	color 1

	
	
	
	21-30
	color 2

	
	
	
	31-40
	color 3

	
	
	
	41-50
	color 4

	
	
	
	51-60
	color 5

	
	
	
	61-70
	color 6

	
	
	
	71-80
	color 7

	
	
	
	81-90
	color 8

	
	
	
	91-100
	color 9

	
	
	
	101-110
	color 10

	
	
	
	111-120
	color 11

	
	
	
	121-130
	color 12

	
	
	
	131-140
	color 13

	
	
	
	141-150
	color 14

	
	
	
	151-160
	color 15

	
	
	
	161-170
	color 16

	
	
	
	171-180
	Full color 

	
	
	
	181-255
	Color gradient from slow to fast

	CH25
	CH18
	Auto Run
	0-7
	No react

	
	
	
	8-131
	Auto mode

	
	
	
	132-249
	Sound control

	
	
	
	250-253
	Reset 

	
	
	
	254-255
	No react


6、After sale service
Our company has long provided customers with responsible technical advice.

If the product fails and needs to be repaired, please show the product warranty card and fill in the relevant content truthfully. At the same time, we also hope that the customer will promptly feedback the product problems to us, so that we can improve the product in the first time.

In addition to lamps, the following accessories can also be selected: Accessories: Options: customer requirements  

(         Warranty card

Customer                       Selling Shop
Company：                     Consign：
Add：                          Tel：

Mob：                                                      

Note: The above blanks must be filled in and stamped truthfully by the end user and the point of sale, otherwise the warranty will not be granted.(Please see below for the warranty period and terms)

Warranty: 1 year since the date bought. (3) Failure or damage caused by force majeure;

Warranty terms: 1. Use this product under normal conditions during the warranty period (4) Failure or damage caused during transportation and unloading;

In the event of failure, you can present this (5) failure or damage caused by failure to follow the instructions for use based on the contents of this warranty clause;

Warranty card and purchase bill (copy), disassemble the product in the designated service center or (6) without the authorization of the manufacturer or non-professionals;

Manufacturers enjoy free maintenance services. (7) Due to the use of a control system that is not approved by the factory or not supported by the factory

2. During the repair period, paid repair services will be implemented in the following situations. Caused by the failure.

(1) A valid warranty card cannot be issued; (8) Consumables (light bulbs).

(2) Omissions, alterations, and no sales shop name on the warranty card;
Note: Based on our company's continuous product improvement, the data contained in this manual may change, and there will be no further notice of changes at that time. The company reserves the right to change the relevant specifications when the product is improved. sorry for the inconveniences caused.










3
3

