	[image: image2.jpg]+/4 amixled AMIXLED.RU, 353 S5 50 55





	[image: image3.jpg]+/4 amixled AMIXLED.RU, 353 S5 50 55







LED PAR 18x18W
[image: image1.png]



User Manual
1. 6 color DMX Channel menu
AH60 DH61

10-channel mode (A001 status)
	Channel
	Function
	Explain

	CH1
	Total dimming
	Total dimming, from dark to bright

	CH2
	R dimming
	R dimming, from dark to bright

	CH3
	G dimming
	G dimming, from dark to bright

	CH4
	B dimming
	B dimming, from dark to bright

	CH5
	W dimming
	W dimming, from dark to bright

	CH6
	Y dimming
	Y dimming, from dark to bright

	CH7
	P dimming
	P dimming, from dark to bright

	CH8
	Total stroboscopic
	Global stroboscopic, from slow to fast

	CH9
	Function selection
	0: Dimming mode; 1－－62: Color selection (selected by ch10); 63—124: Jump change; 125—186: Gradual change; 187—249: Pulse change; 250-255: Voice control (ch10 selects voice control 1 and 2)

	CH10
	Function speed
	Function speed, from slow to fast (color selection for color output, voice control strobe and jump for voice control)


10-channel mode (d001)

	Channel
	Function
	Explain

	CH1
	Total dimming
	Total dimming, from dark to bright

	CH2
	Total stroboscopic
	Global stroboscopic, from slow to fast

	CH3
	Function selection
	0: Dimming mode; 1－－62: Color selection (selected by ch4); 63—124: Jump change; 125—186: Gradual change; 187—249: Pulse change; 250-255: Voice control (ch4 selects voice control 1 and 2)

	CH4
	Function speed
	Function speed, from slow to fast (color selection for color output, voice control strobe and jump for voice control)

	CH5
	R dimming
	R dimming, from dark to bright

	CH6
	G dimming
	G dimming, from dark to bright

	CH7
	B dimming
	B dimming, from dark to bright

	CH8
	W dimming
	W dimming, from dark to bright

	CH9
	Y dimming
	Y dimming, from dark to bright

	CH10
	P dimming
	P dimming, from dark to bright



2. Nixie tube display menu description
	NUM
	content
	Numerical range
	Function and description

	1
	A001
	001- 512
	DMX512 address code setting - 10 channel mode

	2
	d001
	001- 512
	DMX512 address code setting - 10 channel mode

	3
	r255
	000 - 255
	Red dimming, from dark to bright.

	4
	G255
	000 - 255
	Green dimming, from dark to bright.

	5
	b255
	000 - 255
	Blue dimming, from dark to bright.

	6
	u255
	000 - 255
	White dimming, from dark to bright.

	7
	y255
	000 - 255
	Yellow dimming, from dark to bright.

	8
	P255
	000 - 255
	Purple dimming, from dark to bright.

	9
	FH99
	01 - 99
	Stroboscopic, speed from slow to fast.

	10
	CL01
	01 - 62
	Manual color selection mode.

	11
	CC99
	01 - 99
	Jump from slow to fast.。

	12
	DE99
	01 - 99
	Gradient, from slow to fast.

	13
	CP99
	01 - 99
	Pulse changes from slow to fast.

	14
	SU01
	01 - 02
	Mode: 1 is stroboscopic jump, 2 is jump


3. Master slave description

In D001 or A001 state, the slave signal is automatically received without additional setting.
[image: image2.jpg][image: image3.jpg]