CH5Channel description:

| CH | The channel value | Basic function |
| --- | --- | --- |
| 1 | 000-255 | Red lamp bead with linear dimming. |
| 2 | 000-255 | Green lamp bead linear dimming. |
| 3 | 000-255 | Blue lamp bead with linear dimming. |
| 4 | 000-255 | White lamp bead with linear dimming. |
| 5 | 000-255 | The COB lamp bead W tungsten wire effect. |

CH22 Channel Description: Default channel

| CH | The channel value | Basic function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | The COB tungsten wire effect |
| 3 | 000-255 | COB stroboflash |
| 4 | 000-255 | Backlight RGBW dimming |
| 5 | 000-255 | Backlight RGBW strobe |
| 6 | 000-255 | Outer R lamp bead linear dimming. |
| 7 | 000-255 | Outer G lamp bead linear dimming. |
| 8 | 000-255 | Outer B lamp bead linear dimming. |
| 9 | 000-255 | Inner R lamp bead linear dimming. |
| 10 | 000-255 | InnerG lamp bead linear dimming. |
| 11 | 000-255 | Inner B lamp bead linear dimming. |
| 12 | 000-255 | Inner W lamp bead linear dimming. |
| 13 | 000-255 | External RGB effect mode selection |
| 14 | 000-255 | External RGB effect mode speed |
| 15 | 000-255 | Internal RGB effect mode selection |
| 16 | 000-255 | Internal RGB effect mode speed |
| 17 | 000-255 | Internal W effect mode selection |
| 18 | 000-255 | Internal W effect speed |
| 19 | 000-255 | External and internal RGB synchronization effect mode selection |
| 20 | 000-255 | External and internal RGB synchronization effect mode speed |
| 21 | 000-255 | RGBW effect mode selection |
| 22 | 000-255 | The RGBW effect speed |

- Technical parameter:

Voltage: AC100~240V 50 / 60 HZ

Power: 500W (COB: 200W, RGB: 150W, W: 150W)

Light beads: main light: 1 COB custom 1600K color temperature, secondary light: W80 white light 284 RGB light beads

Control mode: DMX512, self-travel, master-slave, voice control, with RDM function.

Channel:CH05, CH22

Dimming: 32bit 0~100% Linear dimming

Features: 24 RGB + 8 W + retro + flash

Operating temperature: -30 degrees ~60 degrees

Floribe frequency: 1-30 HZ

Appearance: metal, black.

Connection mode: DMX512 input / output / power input / output.

IP grade: IP20