User manual

MINI BEAM 350W IP65
Please read the instructions carefully before use

TABLE OF CONTENTS

- Safety Instructions………………………………………………………………………………3

- Technical Specifications…………………………………………………………………………4

- Control Menu ……………………………………………………………………………………5-7

- DMX Channel ……………………………………………………………………………………8-10

- Trouble Shooting…………………………………………………………………………………10

- Fixture Cleaning………………………………………………………… ………………………10

STATEMENT

The product has well capability and intact packing when leave factory. All of  the user should comply with warning item and manual, any misuse cause of the damages are not included in our guarantee, and also can not be responsible  for any malfunction & problem owing to ignore the manual.

1.Safety Instructions

Please keep this User Guide for future consultation. If you sell the unit to another user, be sure that they also receive this instruction booklet.

•Unpack and check carefully there is no transportation damage before using the unit.

•Before operating, ensure that the voltage and frequency of power supply match the power requirements of the unit.

•It’s important to ground the yellow/green conductor to earth in order to avoid electric shock.

•The unit is for indoor use only. Use only in a dry location.

•The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent surfaces. Be sure that no ventilation slots are blocked.

•Disconnect main power before replacement or servicing.

•Make sure there are no flammable materials close to the unit while operating as it is fire hazard.

•Use safety cable when fixes this unit. DO NOT handle the unit by taking its head only, but always by taking its base.

•Maximum ambient temperature is Ta: 40℃. DO NOT operate it where the temperature is higher than this Unit surface temperature may reach up to 85℃. DO NOT touch the housing bare-hand during its operation. Turn off the power and allow about 15 minutes for the unit to cool down before replacing or serving.

•the event of serious operating problem, stop using the unit immediately.Never try to repair the unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please contact the nearest authorized technical assistance center. Always use the same type spare parts.

•DO NOT touch any wire during operation as high voltage might be causing electric shock.

Warning:

•To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

•The housing, the lenses, or the ultraviolet filter must be replaced if they are visibly damaged.

Caution:

•There are no user serviceable parts inside the unit. DO NOT open the housing or attempt any repairs yourself. In the unlikely event your unit may require service, please contact your nearest dealer.

Installation:

•The unit should be mounted via its screw holes on the bracket. Always ensure that the unit is firmly fixed to avoid vibration and slipping while operating. And make sure that the structure to which you are attaching the unit is secure and is able to support a weight of 10 times of the unit’s weight. Also always use a safety cable that can hold 12 times of the weight of the unit when installing the fixture.

•The equipment must be fixed by professionals. And it must be fixed at a place where is out of the touch of people.

2.Technical Specifications

Optics:

Light Source:350W beam

Beam angle: 1.8°

Power:350W

IP rate : IP65

Input voltage:AC100-240V 50-60HZ

Pan movement:540° (16bit )

Tilt movement:270°(16bit)

Color:13+1+colorful rainbow effect+ frost function

Fixed gobo: 13 fixed patterns + white light

Prism: 8 prisms / 8+16 prisms stacked and rotatable, macro function

Dimming system: 0-100% linear adjustment

DMX channels: 16 CH

Features:

XY motor: three-phase electric drive, powerful, faster, more stable and quieter

Control mode: DMX 512, self-propelled, master-slave self-propelled

Support RDM function, built-in program: built-in multiple effect programs

NW:20.7KGS

3.Control Menu

  - Control instructions

1)The function of the 'return' button is to return to the previous interface and exit the function

2)The function of the 'up' button is to select the previous item or increase the value by pressing

for 2 seconds to quickly increase it

3)The function of the "down" button is to select the next item or decrease the value by pressing

for 2 seconds to quickly decrease it

4）The functions of the "OK" button are: confirm save, execute functions, start editing, and

exit editing

  - Interface Description

Two LED indicators on the left

Green - (DMA signal indicator light)

Blue --- (constantly on (self running voice control of the host) flashing (error indication))

The three buttons in the upper right corner are used for language switching and screen rotation.

The sub interfaces corresponding to the four buttons in the bottom row are introduced below.

  - Setting interface

| Option | illustrate |  |
| --- | --- | --- |
| Mode | DMX | Slave state: receiving DMX signals from the control console or host |
|  | AUTO | Host status: automatically running and sending a DMX signal to the slave |
|  | Sound | Host status: Voice controlled operation and sending DMX signal to the slave |
|  | scene | Host status: self programmed to run and send a DMX signal to the slave |
| DMX Address | 1~512 | Press the "OK" button to enter the editing status. The "up" and "down" keys change the address code. Press' OK 'again to exit editing status |
| Channel Mode | 16 | 16CH |
|  | 20 | 20CH |
| X inversion | OFF |  |
|  | ON |  |
| Y inversion | OFF |  |
|  | ON |  |
| X inversion | OFF |  |
|  | ON | Swap channels for the XY axis (including fine tuning) |
| Reset |  | Select 'Yes' to confirm and initiate reset |
| Lamp Control | OFF | Turn off the light bulb |
|  | ON | Turn on the light bulb (please turn it on while the bulb is cool) |
| Lamp On start | OFF | After powering on, the light bulb does not light up (it needs to be manually lit using the menu or console) |
|  | ON | Automatically light up after resetting after power on |
| XY encoder | ON | Using an encoder (optocoupler) to determine out of step and automatically correct the position |
|  | OFF | Correction of position without encoder (optocoupler) |
| NO DMX Signal | KEEP | No signal, continue to maintain the original state |
|  | CLEAR | No signal. The motor returns to its starting position |
| Screensaver | ON | Turn off backlight after idle for 30 seconds |
|  | OFF | lights on |
| Color linear |  | Select 'On' or 'Off' to enable or disable the color wheel linear function |
| Indicator mode | MODE1 | Green with DMX constantly on, blue with host status constantly on |
|  | MODE2 | Green with DMX constantly on, blue with host status constantly on, flashing with error messages |
| Load Default |  | Select 'Yes' to confirm and restore the default settings |

  - Information interface

| Option | illustrate |
| --- | --- |
| Version | Current software version |
| Total Fix | Accumulated use time |
| Fixture time | Usage time since this startup |
| Total Lamp | Accumulated bright bubble time |
| Lamp time | Total bright bubble time |
| Sensor Status | Display the temperature information of the optocoupler hall fan motherboard |

  - Customized interface

This interface is used for self programming and self running programs. When in the DMX mode, there is a DMX signal to view the current data and save the current state

When there is no DMX signal, the desired data can be manually set and saved

| Option | illustrate |  |
| --- | --- | --- |
| RUN STEP | 1-16 | How many steps do I need to run in custom mode |
| NOW STEP | 1-16 | What steps are currently in progress |
| Stop time | 1-99 | Set the current step running time |
| Save |  | Save the current state and proceed to the next step |
| Pan | 0~255 | Press the "OK" button to enter the editing status. Press the "Up" and "Down" keys to change the channel value. Press the 'OK' button again to exit editing status |
| Tilt | 0-255 |  |
| Pan Fine | 0-255 |  |
| Tilt Fine | 0-255 |  |
| …… | 0~255 |  |

  - Advanced interface

| Option | illustrate |
| --- | --- |
| Time zero | Clear the usage time of the bright bubble to zero Retime |
| Lamp control | 2-wire 2-wire control light lighter without half power on/off bubble detection function 5-wire 5-wire control light lighter |
| Calibration | After entering the sub interface, the reset positions of the X-axis, Y-axis, and other motors can be adjusted to compensate for hardware installation errors Attention: Do not perform reset calibration while the motor is running! If the motor is running, please stop it before performing reset calibration If necessary, please perform a reset operation before performing a reset calibration |
| Channel order | Select the channel value to change the function Multiple functions cannot select the same value |
| System Errors | The error message generated by resetting can be cleared by selecting error reset |

4.DMX Channels

16CH

| Channel | Function | Value | Describe |
| --- | --- | --- | --- |
| CH1 | COLOR | 0-4 | White |
|  |  | 5-9 | White + Color1 |
|  |  | 10-14 | Color1 |
|  |  | 15-19 | Color1 + Color2 |
|  |  | 20-24 | Color2 |
|  |  | 25-29 | Color2 + Color3 |
|  |  | 30-34 | Color3 |
|  |  | 35-39 | Color3 + Color4 |
|  |  | 40-44 | Color4 |
|  |  | 45-49 | Color4 + Color5 |
|  |  | 50-54 | Color5 |
|  |  | 55-59 | Color5 + Color6 |
|  |  | 60-64 | Color6 |
|  |  | 65-69 | Color6 + Color7 |
|  |  | 70-74 | Color7 |
|  |  | 75-79 | Color7 + Color8 |
|  |  | 80-84 | Color8 |
|  |  | 85-89 | Color8 + Color9 |
|  |  | 90-94 | Color9 |
|  |  | 95-99 | Color9 + Color10 |
|  |  | 100-104 | Color10 |
|  |  | 105-109 | Color10 + Color11 |
|  |  | 110-114 | Color11 |
|  |  | 115-119 | Color11 + Color12 |
|  |  | 120-124 | Color12 |
|  |  | 125-129 | Color12 + Color13 |
|  |  | 130-134 | Color13 |
|  |  | 135-139 | Color13 + Color14 |
|  |  | 140-144 | Color14 |
|  |  | 145-149 | Color14 + White |
|  |  | 150-199 | Rotate forward (fast to slow) |
|  |  | 200-255 | Rotate reverse (slow to fast) |
| CH2 | STROBE | 0-2 | Dark |
|  |  | 3-50 | Open |
|  |  | 51-240 | Slow strobe to fast strobe |
|  |  | 252-255 | Open |
| CH3 | DIMMER | 0-255 | 0-100% |
| CH4 | GOBO | 0-4 | White |
|  |  | 5-9 | Gobo1 |
|  |  | 10-14 | Gobo2 |
|  |  | 15-19 | Gobo3 |
|  |  | 20-24 | Gobo4 |
|  |  | 25-29 | Gobo5 |
|  |  | 30-34 | Gobo6 |
|  |  | 35-39 | Gobo7 |
|  |  | 40-44 | Gobo8 |
|  |  | 45-49 | Gobo9 |
|  |  | 50-54 | Gobo10 |
|  |  | 55-59 | Gobo11 |
|  |  | 60-64 | Gobo12 |
|  |  | 65-69 | Gobo13 |
|  |  | 70-129 | Rotate reverse (fast to slow) |
|  |  | 130-190 | Rotate forward (slow to fast) |
|  |  | 191-195 | Shake slow to fast Gobo1 |
|  |  | 196-200 | Shake slow to fast Gobo2 |
|  |  | 201-205 | Shake slow to fast Gobo3 |
|  |  | 206-210 | Shake slow to fast Gobo4 |
|  |  | 211-215 | Shake slow to fast Gobo5 |
|  |  | 216-220 | Shake slow to fast Gobo6 |
|  |  | 221-225 | Shake slow to fast Gobo7 |
|  |  | 226-230 | Shake slow to fast Gobo8 |
|  |  | 231-235 | Shake slow to fast Gobo9 |
|  |  | 236-240 | Shake slow to fast Gobo10 |
|  |  | 241-245 | Shake slow to fast Gobo11 |
|  |  | 246-250 | Shake slow to fast Gobo12 |
|  |  | 251-255 | Shake slow to fast Gobo13 |
| CH5 | PRISM1 | 0-127 | None |
|  |  | 128-255 | Insert prism1 |
| CH6 | PRISM1 ROT | 0-127 | 0-400 degrees |
|  |  | 128-191 | Rotate reverse (fast to slow) |
|  |  | 192-255 | Rotate forward (slow to fast) |
| CH7 | PRISM2 | 0-127 | None |
|  |  | 128-255 | Insert prism2 |
| CH8 | FROST | 0-127 | None |
|  |  | 128-191 | Insert Frost |
|  |  | 192-255 | Insert Colorful |
| CH9 | FOCUS | 0-255 | far to near |
| CH10 | PAN | 0-255 | 0-540 |
| CH11 | PAN Fine | 0-255 |  |
| CH12 | TILT | 0-255 | 0-270 |
| CH13 | TILT Fine | 0-255 |  |
| CH14 | XY SPEED | 0-255 | fast to slow |
| CH15 | RESET | 26-76 | Reset EFT over 5 seconds |
|  |  | 77-127 | Reset XY over 5 seconds |
|  |  | 128-255 | Reset ALL over 5 seconds |
| CH16 | LAMP | 10-100 | Lamp off over 5 seconds |
|  |  | 101-255 | Lamp on over 5 seconds |

5.Trouble Shooting

Following are a few common problems that may occur during operation. Here are some suggestions for easy troubleshooting:

A. The unit does not work, no light and the fan does not work

1. Check the connection of power and main fuse.

2. Measure the mains voltage on the main connector.

3. Check the power on LED.

B. Not responding to DMX controller

1. DMX LED should be on. If not, check DMX connectors, cables to see if link properly.

2. If the DMX LED is on and no response to the channel, check the address settings and DMX polarity.

3. If you have intermittent DMX signal problems, check the pins on connectors or on

PCB of the unit or the previous one.

4. Try to use another DMX controller.

5. Check if the DMX cables run near or run alongside to high voltage cables that may cause damage or interference to DMX interface circuit.

C. One of the channels is not working well

1. The stepper motor might be damaged or the cable connected to the PCB is broken.

2. The motor’s drive IC on the PCB might be out of condition

6.Fixture Cleaning

The cleaning of internal and external optical lenses and/or mirrors must be carried out periodically to optimize light output. Cleaning frequency depends on the environment in which the fixture operates: damp, smoky or particularly dirty surrounding can cause greater accumulation of dirt on the unit’s optics.

 Clean with soft cloth using normal glass cleaning fluid.

 Always dry the parts carefully.

 Clean the external optics at least every 20 days. Clean the internal optics at least every 30/60 days.