LED 96+32 RGBW STROBE

Instruction Manual

Definition of display panel and buttons

Menu keys：Enter/Return to Main Menu

Up Key: Menu function up selection/parameter increment

Down key: Menu function down selection/parameter decrement

Confirm key：Select the menu function and make a confirmation and save when modifying the menu function parameters.

- Menu functions

| Main menu | Submenu | Menu Function Description |  |
| --- | --- | --- | --- |
| System settings | DMA address | Modify the DMA address code up or down (with an optional range of 001-512), confirm to save, and default to 001. |  |
|  | Advanced Settings | Modify the encoder on/off up or down, save with the confirm button, and default to on. |  |
| Operating mode | Operating mode | Select the operation mode up or down, such as DMA/voice control/automatic, and press the confirm button to save. The default mode is DMA. |  |
|  | Channel selection | Select channels CH4/CH17/CH121/CH145 up or down to save, default to CH17. |  |
| Manual | Dimming | Modify the dimming parameter values up or down (adjustable range 000-255), confirm to save, default to 255. |  |
|  | RGBW strobe | Modify the strobe parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Red | Modify the red parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Green | Modify the green parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Blue | Modify the blue parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | White | Modify the white parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | RGB mode | Modify the mode parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | RGB speed | Modify the speed parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Seven Colors | Modify the seven color parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | W mode | Modify the W mode parameter values up or down (adjustable range 000-255), confirm to save, default to 000 |  |
|  | W speed | Modify the W speed parameter value up or down (adjustable range 000-255), confirm to save, default to 000 |  |
|  | RGBW mode | Modify the parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | RGBW speed | Modify the parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Background color | Modify the parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Background tone light | Modify the parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Character | Modify the character parameter values up or down (adjustable range 000-255), confirm to save, default to 000. |  |
|  | Direction | Modify the direction parameter value up or down (adjustable range 000-255), confirm to save, default to 000. |  |
| Display Settings | Language selection | Select Chinese/EN (English) up or down, confirm to save, default to Chinese. |  |
|  | Invert Settings | Select up or down to turn off/on the inverted display setting, confirm to save, default to off. |  |
|  | Screen saver | Select up or down to turn off/on screen saver, confirm to save, default to on. |  |
|  | Software version | G37A program identification code, cannot be modified. |  |
| Factory settings | Please enter password for display! Change up or down to 138, then press the confirm button to enter the factory settings。 | Red | Modify the red light bead current (032-255) up or down, confirm to save, default to 255. |
|  |  | Green | Modify the green light bead current (032-255) up or down, confirm to save, default to 255. |
|  |  | Blue | Modify the blue light bead current (032-255) up or down, confirm to save, default to 255. |
|  |  | White | Modify the red light bead current (032-255) up or down, confirm to save, default to 255. |
|  |  | Logo | Modify the green bead current (032-255) up or down, confirm to save, default to 255 |
|  |  | Temperature | Modify the blue light bead current (032-255) up or down, confirm to save, default to 255 |
|  |  | Temperature - Fan | Modify the white light bead current (032-255) up or down, confirm to save, default to 255 |

CHANNEL

CH4：

| Thoroughfare | Channel value | Basic function |
| --- | --- | --- |
| 1 | 000-255 | The red light bead dims linearly. |
| 2 | 000-255 | Green bead linear dimming. |
| 3 | 000-255 | Blue bead dimming linearly. |
| 4 | 000-255 | White light beads linearly dimming. |

CH17

| Thoroughfare | Channel value | Basic function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | RGBW strobe |
| 3 | 000-255 | The red light bead dims linearly. |
| 4 | 000-255 | Green bead linear dimming. |
| 5 | 000-255 | Blue bead dimming linearly. |
| 6 | 000-255 | White light beads linearly dimming. |
| 7 | 000-255 | Colorful (walking through the mode will have a colorful effect) |
| 8 | 000-255 | RGB effect mode selection |
| 9 | 000-255 | RGB effect mode speed |
| 10 | 000-255 | W Effect Mode Selection |
| 11 | 000-255 | W effect speed |
| 12 | 000-255 | RGBW effect mode selection |
| 13 | 000-255 | RGBW effect speed |
| 14 | 000-255 | Background color |
| 15 | 000-255 | Background tone light |
| 16 | 000-255 | Static pattern (numbers/letters) |
| 17 | 000-255 | Static pattern direction. |

CH145

| Thoroughfare | Channel value | Basic function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | RGBW strobe |
| 3 | 000-255 | The red light bead dims linearly. |
| 4 | 000-255 | Green bead linear dimming. |
| 5 | 000-255 | Blue bead dimming linearly. |
| 6 | 000-255 | White light beads linearly dimming. |
| 7 | 000-255 | RGB effect mode selection |
| 8 | 000-255 | RGB effect mode speed |
| 9 | 000-255 | Colorful (walking through the mode will have a colorful effect) |
| 10 | 000-255 | W Effect Mode Selection |
| 11 | 000-255 | W effect speed |
| 12 | 000-255 | RGBW effect mode selection |
| 13 | 000-255 | RGBW effect speed |
| 14 | 000-255 | Background color |
| 15 | 000-255 | Background tone light |
| 16 | 000-255 | Static pattern (numbers/letters) |
| 17 | 000-255 | Static pattern direction. |
| 18 | 000-255 | W1 |
| 19 | 000-255 | W2 |
| 20 | 000-255 | W3 |
| 21 | 000-255 | W4 |
| 22 | 000-255 | W5 |
| … | … … | … … |
| 47 | 000-255 | W32 |
| 48 | 000-255 | RGB1 |
| 49 | 000-255 | RGB2 |
| 50 | 000-255 | RGB3 |
| … | … … | … … |
| 145 | 000-255 | RGB96 |

Technical parameters:

Voltage: AC100~240V 50/60HZ

Power: 350W (RGB: 130W, W: 220W)

Lamp beads: 256 3030 white light 1 watt, 864 5050 RGB 0.2 watt lamp beads

Control mode: DMX512, self-propelled, master-slave, voice control, with RDM function.

Channels: CH04, CH17, CH145

Dimming:  0-100% linear dimming

Features: 96 segments RGB+32 segments W+dyeing+flashing

Working temperature: -30 °C~60 °C

Strobe frequency: 1-30Hz

Appearance: Metal, black

Connection method: DMX512 input/output/power input/output.

IP level: IP20