Mini 6 Retro Light

Operating Manual

1. Display Panel

Menu confirms up and down

Key definition:

Order key: selection function

Up key:, the parameter add

Down key:, parameter parameter

Confirmation key: determine and save

2. Menu Function

After power on, press the menu button to display the menu menu; modify the function parameters up or down to save the current function and parameters (with power loss memory after saving).

menu function table:

| Manu | Submenu 1 | Submenu 2 | remarks |
| --- | --- | --- | --- |
| Addr | D001/D512 |  | address code |
| CHAL | CH04/CH12/CH24 |  | channel |
| AUTO | CA50 | 01-50 | Since go |
|  | CB50 | 01-50 |  |
|  | CC50 | 01-50 |  |
|  | CD50 | 01-50 |  |
|  | CE50 | 01-50 |  |
|  | CF50 | 01-50 |  |
|  | CG50 | 01-50 |  |
|  | CH50 | 01-50 |  |
|  | CJ50 | 01-50 |  |
|  | … |  |  |
|  | DY | 01-50 |  |
| S OUD | SO01-SO99 |  | Sound control mode |
| STAT | c000 | c000-0255 | Operation mode, a total of 100 operating effects |
|  | r000 | r000-r255 | Red manual dimming |
|  | g000 | g000-g255 | Green manual dimming |
|  | b000 | b000-b255 | Blue manual dimming |
|  | w000 | w000-w255 | White manual dimming |
|  | t000 | t000-t255 | stroboflash |
| Temp | C 000 | T085 | thermoregulation |
| Rest | On/Off |  | factory data reset |

3. Factory Setting

From A 001 to A 512, press the menu button for 5 seconds to enter the factory setting. The factory is mainly set for the functions of output power of lamps, fan setting mode, setting temperature protection point and sending parameters. The factory sets any mode and presses the menu button for 5 seconds.

Factory Settings Table:

| R255 |  | R032 | Modify the red bead current (R032~ R255), confirm the key is saved, default R220. |
| --- | --- | --- | --- |
| G255 |  | G032 | Modify the green bead current (G032~G255), confirm the key save, default G220. |
| B255 |  | B032 | Change the blue bead current (B032~ B255), confirm the key save, default B220. |
| W255 |  | W032 | Modify the white lamp bead current up or down (w 032~ w 255), confirm that the key is saved, default w 220. |
| Dir0 |  | Dir7 | Modify the flow direction up or down (d 0~ d7), save the key, default d0. |
| FAN0 |  | FAN1 | Fan setting: the FAN 0 lamp bead light will start the fan, the FAN 1 temperature reaches 40 degrees to start the fan, confirm the key to save. |
| T040 |  | T 070 | Set the temperature protection point, modify the parameters up or down (40℃ ~70℃), press the confirmation key to save, default T065. |
| S end |  | S end | Send the factory setting parameters up or down to all other lamps connected in parallel with three-core signal lines; confirm the sending parameters, press the dish order key for 5 seconds to exit, and press the confirmation key to cancel the sending. |

4. Master-slave Control

Two or more identical lamps are connected with DMX three-core signal line, all lamps are set as A001~A512 any address code, any one is set as the host, other lamps are slave, all slave display screens are not flashing; when the host adjusts the gradient, pulse, voice control and self-walking effect, all slave synchronous gradual, ripple, voice control and self-walking effect.

Special attention: 1.a group of lamps can only be set with one host, if there are more than one host, all the lamps will flash and not synchronized.

2.All lamps must be used when the DMX512 console is closed.

5. DMX512 Channels

After power-on, all the lamp address codes are set, and then all the lamps are connected to the DMX512 console in parallel with three-core signal lines. The address code will stop flashing, indicating that the DMX512 console signal has been sent to the lamp, and the relevant functions are controlled with the DMX512 console according to the instructions of the channel table.

4CH Description:

| CH4 | Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Red total dimming |
| 2 | 000-255 | Green total dimming |
| 3 | 000-255 | Blue total dimming |
| 4 | 000-255 | White total dimming |

12CH Description:

| CH12 | Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | stroboflash |
| 3 | 000-255 | pigment |
| 4 | 000-255 | Effect mode (106 effects), 255 is voice control |
| 5 | 000-255 | velocity |
| 6 | 000-255 | White light screen flash |
| 7 | 000-255 | White light effect mode (73 effects) with lower priority than voice control |
| 8 | 000-255 | White light speed |
| 9 | 000-255 | Red lamp bead background linear dimming |
| 10 | 000-255 | Green lamp bead background linear dimming |
| 11 | 000-255 | Blue lamp bead background linear dimming |
| 12 | 000-255 | White lamp bead background linear dimming |

144CH Description:

| CH24 | Value | Function |
| --- | --- | --- |
| 1 | 000-255 | R segment 1 red lamp bead linear dimming |
| 2 | 000-255 | G Linear dimming in paragraph 1 |
| 3 | 000-255 | B Linear dimming in segment 1 |
| … | … | … … |
| 16 | 000-255 | R segment 6 red lamp bead linear dimming |
| 17 | 000-255 | G Linear dimming in paragraph 6 |
| 18 | 000-255 | B The linear dimming of the blue lamp beads in segment 6 |
| 19 | 000-255 | W linear dimming of white lamp beads in paragraph 1 |
| … | … | … … |
| 24 | 000-255 | W segment 6 white lamp bead linear dimming |

- Technical Parameters

- Voltage: AC100 ~ 220V 50 / 60HZ

Power: 144 * 0.3W + 6 * 50W

Beads: 144 5050 three-color LED beads + 6 warm white COB beads

Control mode: DMX512, self-walking, master-slave, voice control, with RDM function.

Channels: CH04, CH12, CH24

Dimming: 32 bit 0 ~ 100% linear dimming

Features: dyeing + explosion flash + auxiliary light

Operating temperature: -30 degrees ~ 50 degrees

Strobe frequency: 1 ~ 30 HZ

Appearance: metal, black

Connection mode: DMX512 I / O / Power I / O.   IP Level: IP20

7.Product Size and Packing

Product Size:32*19.5*114cm

Inside Carton Packing Size:118*21*21cm

Outside Carton Packing Size:120*45*24（2in1）

Net Weight:6.05kg

Gross Weight :16.3kg (2in1)

Power Cable*1, Signal Cable*1, Manual*1