USER MANUAL

CONTENTS

1

Chapter 1 Installation and attention

1

1.
Maintenance

1

2.
Statement

1

3.
Safety Precaution

1

4.
Cable connection（DMX）

2

5.
Rigging (Optional)

3

6.
RDM NOTE

4

Chapter 2  Panel operation

4

1.
Brief

4

2.
Operation

4

1.
Operate fixture with touch or encoder/button

4

2.
Parameter value setting

5

3.
Boolean parameter setting

5

4.
Sub Menu（Parameter）

6

3.
Operation and parameter instruction

6

1.
DMX Address setting

6

2.
Fixture operating mode setting

7

3.
Set display

7

4.
Scene

8

5.
Set light run parameter

9

6.
Status and information

11

Chapter 3  Channel description

11

1.
Channel table



Chapter 1 Installation and attention 

Maintenance

To reduce the risk of electrical shock or fire, do not expose this unit to rain or moisture.

Intermittently using will extend this item’s service life.

Please clear the fan, fan net, and optical lens in order to keep good work state.

Do not use the alcohol or any other organic solvent to wipe the shell.

Statement

The product has perfect performance and integrity packing. All users should be strictly complying with the warning and operating instructions as stated. Or we aren’t in charge of any result by misusing. Any damage resulting by misuse is not within the Company’s warranty. Any fault or problem caused by neglecting the manual is also not in the charge of dealers.

   Note: All information is subject to change without prior notice.

Safety Precaution
In order to guarantee the product’s life, please don’t put it in the damp places or even the environment over 60degress.

Always mount this unit in safe and stable matter.

Install or dismantle should operate by professional engineer.

Using lamp，the change rate of power voltage should be within±10%，If the voltage is too high，it will shorten the light’s life; If it’s not enough, will influence the effect.

Please restart it 20 minutes later after turning off light , until full-cooling. Frequent switching will reduce the life span of lamps and bulbs; intermittent using will improve the life of bulbs and lamps.

In order to make sure the product is used well, please read the Manual carefully.

Cable connection（DMX）

Use a cable conforming to specifications EIA RS-485: 2-pole twisted, shielded, 120Ohm characteristic impedance, 22-24 AWG, low capacity. Do not use microphone cable or other cable with characteristics differing from those specified. The end connections must be made using XLR type 3 or 5-pin male/female connectors. A terminating plug must be inserted into the last projector with a resistance of 120Ohm (minimum 1/4 W) between terminals 2 and 3.
IMPORTANT: The wires  must not make contact with each  other or with the metal casing of the connectors. The casing itself must be connected to the shield braid and to pin 1 of the connectors.[image: image1.jpg]® DMX5120
5PINe

Sy e
Y + s

o

t

DMX5120

o
DMX512¢:
3PN

| S l@ | screens
soucr





Figure 1  DMX Cable connection

Rigging (Optional)

This equipment can be positioned and fixed by clamp in every direction of the stage. Locking system makes it easy to fasten to the bracket. 

Attention! Two clamps is needed to fix the equipment. Every clamp is locked by fastener of 1/4 kind. Fastener can only be locked clockwise.

Attention! Fasten a safety string to the additional hole of side aluminum piece. The secondary accessory can not hang on the delivery handle. Nip the equipment on bracket.

Check if rigging clamp (not including the one inside) damaged or not? If stand ten times weight as the equipment. Make sure the architecture can stand ten times weight as all the equipments, clamps, wirings and other additional fixtures.
Screws for clamping must be fixed firmly. Take one M12 screw (Grade 8.8 or higher) to clamp bracket, and then screw the nuts.
Level the two hanging points at the bottom of clamp. Insert fastener to the bottom, lock the two levers by 1/4 rotating clockwise; then install another clamp.
Install on safety string which stands at least ten times weight as equipment. Terminal of the accessory is designed for clamps.
Make sure pan/tilt lock unlocked or not. Keep the distance more than 1M from equipment to flammable material or lighting source.
[image: image2.png]


[image: image3.png]



Figure 2  Installation 

RDM Note

RDM is an extended version of DMX512-A protocol. It is a remote device management protocol. Traditional DMX512 protocol communication is one-way communication. The protocol is based on RS-485 bus. RS-485 is a time-sharing multi-point, half-duplex protocol. Only one port is allowed to output at the same time. So, when using RDM, we should pay attention to it. The following points:

To use console or host device that supports RDM host protocol.

Use bidirectional signal amplifier, traditional one-way signal amplifier is not suitable for RDM protocol, because the RMD protocol needs feedback data, the use of one-way amplifier will block the return of data, resulting in no search fixture;

All fixture must be set to DMX mode to ensure only one host on the cable.

A 120 ohm impedance matching resistor must be inserted between terminals 2 and 3 of the terminal plug. When the signal line is longer, reducing the signal reflection will make the differential signal more stable and beneficial to the quality of communication.

When the fixture appears to accept DMX control, but can not been search by RDM host, first check the signal amplifier, and then check whether the signal line 2, 3 lines have bad contact.

Chapter 2  Panel operation

Brief

The light panel diagram show as Figure 3, above area is fixture description, below area show fixture real-time status, such as DMX cable status, lamp status, error or information(ps. when there are message hav’t been checked, echo ’ERR’ in status bar, otherwise echo ‘NOR’).

Display & operation just like ‘Android operation system’, when select or set item value, system save the settting immediately. 

RDM protocol is embed in fixture, user set DMX address via cable using the controller surpport RDM function. when fixture was search by controller, displayer will echo ‘RDM’ indicate this RDM is work. 

[image: image4.jpg]1BEHR =~





Figure 3  Panel diagram

Operation

Operate fixture with touch or encoder/button

The left area is TFT Displayer and touch(product which support touch), chick item or value with finger will to complete operation of set light setting(parameters) or view light state.

The area on the right hand side is rotary encoder with button or key, As auxiliary input interface, if fixture disable touch function, the encoder/key can been choose to set or view the item, and then press the encoder button/key to confirm the selection, rotary encoder or push key again set the parameter value, finally, Press encoder button/key one again to save value or setting.

Parameter value setting

When the selected item is value need to been modified, the dialog shown in Figure 4 will popup.

[image: image5.jpg]But no save

w008

Apply Value to system

—>

<«—Adjust Value quick

:) Adjust Value

Save value & quit




Figure 4  Dialog of value setting

Modify value：Can quickly modify value via pull the slider to the desired position, or click  the button of ‘up’ or ‘down’ whit finger on the right side to set the exact desired value, another way is roll encoder on the right hand side of panel.

Apply value：When Value had been modified, Then press the bottom of ‘apply’ in the left corner to apply to the light, but hav’t saved；

Save Value：Any time, click on the lower right corner of the "OK" button, the setting will been saved into internal memory.

Boolean parameter setting

when the selected parameters is a Boolean value (such as ON or OFF), can directly modify setting by chick corresponding item, the setting will been saved right now. 

When the parameter is a key item, chick corresponding item, a dialog shown in Figure 5 will been popup ask for the confirm. Chick ‘sure’ to confirm.

[image: image6.png]



Figure 5  Dialog of confirm

Sub Menu（Parameter）

[image: image7.jpg]001

Preu |

Hext»

bl

gl
Gz,Tilth
83.Pan Fine
64.Tilt Fine 000

H/S ChQQSE“

Tllt ﬂFFsetg,f

bata hold

DHaA IndlCatE«
Screen Light

}Nndeg
a8

FlXtUtg
U9F519n"1

Light timéffwm

iotal time





Figure 6  Parameter menu

Operation and parameter instruction

Chick item of main menu, enter corresponding sub menu shown in Figure 6, In main menu, chick 1/6 function button into corresponding parameter menu.

In sub menu(page), chick main item on the left side of displayer, can shift to corresponding sub menu(page) quickly. 

DMX Address setting

Enter page show in Figure6-1, can set fixture DMX address, channel mode and so on.

The menu settings of fixture have optimized the setting of addresses. Several settings of the address are as follows:

Select " Prev " or "Next", the fixture will be based on the current address and channel mode, automatically calculate the next or last address, make address setting can quickly;

Click on the address value, you can enter the numeric editing window, where you can set any valid address, fixture system automatically get the current number of channels, automatically filter the unusable address (512 - the current number of channels).

Fixture support RDM protocol, remote address can be set through RDM.

Provide one buttons:

Channel mode’chan’: you can choose different channel modes by cycle.

Fixture operating mode setting

Through the page shown in Figure 6-2, the operating mode of the fixture can be set and the lamp can be controlled. The fixture supports four operating modes (DMX mode, auto mode, voice control mode and scene mode). Detailed parameter settings can be refer in the previous section. Specific parameter descriptions are as follows:

operating mode

	DMX Ctrl
	DMX mode, receive DMX signal, RDM signal

	Auto Run
	Fixture run automatically according to built-in programs

	Sound Ctrl
	When the fixture detects a strong sound, the fixture automatically runs a scene according to the built-in program, otherwise it will stay the last scene

	Scene Mode 01
	runs in a set scebe, which supports most of the custom editing of 10 scenes.

	
	1~10
	outputs the specified scene

	
	Auto
	Automatically loops the output scene in the set scene time (non-zero) order, and the scene with time 0 automatically ignore

	M/S Choose
	Master and slave selection, non-DMX mode takes effect, select the mode of data output, fixture detect DMX cable state automatic switch output, prevent data conflicts

	
	Master
	fixture runs built-in program. If DMX has no signal, it outputs data (synchronization), otherwise it does not output data.

	
	Slave
	ixture runs built-in program and do not output data

	
	Auto
	If DMX has no signal, the fxiture will runs built-in program. Otherwise, the fixture will run in DMX Mode(follow DMX).

	Lamp switch
	(Lamp light source) pop-up confirmation dialog box, select "SURE" to confirm the current operation, turn on or off the lamp, switch time interval limited to 30 seconds

	
	Off
	the current lamp output is off

	
	On
	The current lamp output is turned on


Scene mode applies to a single or a small number of fixture, just output a fixed scene, or need to run a simple program, you no need connect to the console, in the scene page can be edited.

If the light source is lamp, wait for 10 minutes before turning off the lamp.

Set display

The fixture support Chinese and English, invert display and so on. Enter the corresponding parameter settings as shown in Figure 6-3. The specific menu contents are as follows:

DISPLAY SETTING

	Language
	display language settings

	
	English
	English display

	
	Chinese
	Chinese display

	Screen saver
	Set screen 30 seconds without operation, the screen's display content or method.

	
	OFF
	Keep the last operation page

	
	Mode1
	Black

	
	Mode2
	Black screen, showing the address code of the current fixture in the lower left corner.

	
	Mode3
	Display trademark information, address code and operation mode.

	Screen Rot
	Set the display direction of the screen.

	
	OFF
	No reverse display

	
	ON
	Reverse display

	DMX Indicate
	Set the indication mode of DMX signal indicator.

	
	Mode1
	When signal is bright, no signal is off.

	
	Mode2
	When signal is off, no signal is bright.

	
	Mode3
	When signal is flash, no signal is off.

	Screen Lihgt
	Set the screen backlight for 10 seconds without operation

	
	1~10
	10 


Scene

Enter the page shown in Figure 6-4, and the fixture enters the scene editing mode. Under this page, the fixture does not receive DMX console data, and the edited data will effect on the fixture immediately.

The content of the page depends on the currently selected channel mode, and the channel content and order displayed are consistent with the fixture channel table. Through this page, you can edit 10 scenes, as shown in the following table:

SCENE MODE

	Scene Select
	Select the current operation scenario.

	
	1~10
	The 10 scenes sets the format

	Scene Time
	Sets the retention time of the current scene when it is automatic, unit in 0.1 seconds.

	
	0
	The current scene is not output in automatic scene output.

	
	1-255
	0..1s-25.5s

	1. PAN
	0-255
	Set up the data of each channel, and the contents and order of the display are one-to-one correspondence with the channel list of fixture.

	……
	0-255
	

	……
	0-255
	

	N. Function
	0-255
	


If the reset channel in the scene edits the effective reset data, the fixture will reset, but after reset, the corresponding reset channel value will automatically set 0, preventing multiple consecutive resets.

Looking at this page, you can get the current channel table slot of the fixture. For specific channel data, please refer to the detailed channel description.

Set light run parameter

Enter the page shown in Figure 6-5, adjust the field parameters of fixture, facilitate the installation of fixture, etc.

ADVANCED SETTING

	Pan Invert
	Set the rotation direction of PAN

	
	OFF
	

	
	ON
	

	Tilt Invert
	Set the rotation direction of TILT

	
	OFF
	

	
	ON
	

	P/T Rectify
	Setting up fixture to detect XY lost step and correct

	
	OFF
	Uncorrected position after out of step

	
	ON
	After losing step, the position is automatically corrected and the out of step fault is recorded.

	Pan Offset
	Setting the zero point of the PAN of the fixture

	
	4-150
	　

	Tilt Offset
	Setting the zero point of the TILT of the fixture

	
	4-48
	　

	Data hold
	When the fixture is not equipped with DMX signal, the output state of the fixture

	
	OFF
	No signal, so the motor and light source return to the position and state when reset is completed.

	
	ON
	No signal, keep the last frame DMX data output.

	Lamp mode
	(lamp light source) Set the way to first open the lamp after power up

	
	Power on
	Turn on the lamp at power up and reset the lamp after 30 seconds.

	
	After reset
	Reset the fixture after 3 seconds when power-on, and turn on the lamp after reset.

	
	Manual
	After reset, manually turn on the lamp through the menu or console.

	Reset
	Reset fixture

	Factory Setting
	Pop up the confirmation box, select "SURE", and return the lamp parameters to the factory settings.


When choosing power-on mode, the lamp will wait for 30 seconds after power-on, let the lamp fully start, internal voltage is stable enough, then start the reset program, if the field capacity is stable, recommend power-on mode.

When the fixture can not calibrate the position, please check whether the "P/T Rectify" is turned off.

When the signal is unplugged, check the Data Hold setting first if the position of the fixture is not output as expected.

When setting the XY offset, after setting up, please control XY with the maximum stroke first to check that XY will not bump into the positioning rod or shell.

Status and information

Entering the page shown in Figure 6-6, you can view the information and real-time status of the fixture to get their usage status. If the fixture need customer service, please provide the status information displayed on the page as a basis for judgment, as shown in the following table:

STATUS INFORMATION

	Stepper info
	Display information status of all motors and signals in fixture.

	
	Hall
	No display, indicating that the motor has no Hall, 0 indicating that the motor leaves the correction position point, 1 indicating that the motor is in the correction position point

	
	Status
	Display motor reset status

	
	PAN
	Display real-time position value of PAN optocoupler feedback

	
	TILT
	Display real-time position value of TILT optocoupler feedback

	
	PAN OP
	Displays the PAN TILT optocoupler two signal level state, binary

	Error Logging
	Show the latest 8 error records when the fixture is reset and running. The error records are not saved after power failure. The current power cycle is valid.

	
	Error Logging
	Total number of failures detected after power on

	
	12：:03
	The time of power failure when the fault occurs is in minutes.

	
	Hall error
	The effective hall signal is not detected when the motor is reset

	
	Hall short
	When the motor is reset, the hall signal of the motor is always effective

	
	Opti error
	No effective optocoupler signal is detected when the motor is reset.

	
	Lose stop
	The corresponding motor is out of step during its operation.

	
	Hit
	Striking the positioning rod when the motor is reset

	
	Lamp error
	Lamp explosion accident

	
	NTC error
	The temperature sensor signal is abnormal

	
	Fan error
	The main fan is not working properly.

	Fixture status
	Displays the critical state data of the current fixture for reference.

	
	Communication prec
	0~100%, Communication quality of internal data link of lamps and lanterns

	
	Error cnt
	The number of erroneous frames was detected after power on, and the total number of erroneous frames was detected.

	
	Light Temperature
	Show the temperature of the current light source, "---" means no detection.

	
	Panel Temperatrue
	Displays the temperature of the current display panel or the ambient temperature.

	
	Sensor1 Temperatrue
	Display the ambient temperature of the motherboard temperature or the motherboard installation position.

	Version
	Display the information and version of the current fixtrue, important reference for after sales maintenance.

	
	Device
	The name of the fixture is the same as the equipment information of RDM.

	
	Model
	The type of fixture is the same as the model information of RDM.

	
	Panel
	Firmware version and serial number of display panel

	
	Main Board
	Firmware version and serial number of mother board 1

	Light time
	Record the total cumulative time of light source opening, unit minute, user manual cleaning, as a reference for regular maintenance of light source time

	Total time
	The total accumulated time for recording the opening of fixture is not allowed to be removed.


Chapter 3  Channel description

Channel table

This luminaire channel can be viewed in scene mode in order, channel mode is set in the "Address Settings" page, specific details of the data as follows:
CHANNEL TABLE

	 LIST-1
	 NAME
	 VALUE
	 BRIEF

	 [ CH1 ]
	 Colour
	
	

	 
	 
	 0-4
	 White

	 
	 
	 5-9
	 White+colour1

	 
	 
	 10-14
	 Colour1

	 
	 
	 15-19
	 Colour1+Colour2

	 
	 
	 20-24
	 Colour2

	 
	 
	 25-29
	 Colour2+Colour3

	 
	 
	 30-34
	 Colour3

	 
	 
	 35-39
	 Colour3+Colour4

	 
	 
	 40-44
	 Colour4

	 
	 
	 45-49
	 Colour4+Colour5

	 
	 
	 50-54
	 Colour5

	 
	 
	 55-59
	 Colour5+Colour6

	 
	 
	 60-64
	 Colour6

	 
	 
	 65-69
	 Colour6+Colour7

	 
	 
	 70-74
	 Colour7

	 
	 
	 75-79
	 Colour7+Colour8

	 
	 
	 80-84
	 Colour8

	 
	 
	 85-89
	 Colour8+Colour9

	 
	 
	 90-94
	 Colour9

	 
	 
	 95-99
	 Colour9+Colour10

	 
	 
	 100-104
	 Colour10

	 
	 
	 105-109
	 Colour10+Colour11

	 
	 
	 110-114
	 Colour11

	 
	 
	 115-119
	 Colour11+White

	 
	 
	 120-187
	 Rotate forward (fast to slow)

	
	
	 188-255
	 Rotate reverse (slow to fast)

	 [ CH2 ]
	 Strobe
	
	

	 
	 
	 0-3
	 Dark

	 
	 
	 4-103
	 Pluse strobe slow to fast

	 
	 
	 104-107
	 Open

	 
	 
	 108-207
	 Fade strobe slow to fast

	 
	 
	 208-212
	 Open

	 
	 
	 213-251
	 Rand strobe slow to fast

	 
	 
	 252-255
	 Open

	 [ CH3 ]
	 Dimmer
	 0-255
	 0-100% dimmer

	 [ CH4 ]
	 Gobo
	
	

	 
	 
	 0-4
	 White

	 
	 
	 5-9
	 Gobo1

	 
	 
	 10-14
	 Gobo2

	 
	 
	 15-19
	 Gobo3

	 
	 
	 20-24
	 Gobo4

	 
	 
	 25-29
	 Gobo5

	 
	 
	 30-34
	 Gobo6

	 
	 
	 35-39
	 Gobo7

	 
	 
	 40-44
	 Gobo8

	 
	 
	 45-49
	 Gobo9

	 
	 
	 50-54
	 Gobo10

	 
	 
	 55-59
	 Gobo11

	 
	 
	 60-64
	 Gobo12

	 
	 
	 65-69
	 Gobo13

	 
	 
	 70-74
	 Gobo14

	 
	 
	 75-79
	 Gobo15

	 
	 
	 80-84
	 Gobo16

	 
	 
	 85-89
	 Gobo17

	 
	 
	 90-128
	 Rotate reverse (fast to slow)

	 
	 
	 129-131
	 Stop

	 
	 
	 132-170
	 Rotate forward (slow to fast)

	 
	 
	 171-175
	 Shake slow to fast Gobo1

	 
	 
	 176-180
	 Shake slow to fast Gobo2

	 
	 
	 181-185
	 Shake slow to fast Gobo3

	 
	 
	 186-190
	 Shake slow to fast Gobo4

	 
	 
	 191-195
	 Shake slow to fast Gobo5

	 
	 
	 196-200
	 Shake slow to fast Gobo6

	 
	 
	 201-205
	 Shake slow to fast Gobo7

	 
	 
	 206-210
	 Shake slow to fast Gobo8

	 
	 
	 211-215
	 Shake slow to fast Gobo9

	
	
	 216-220
	 Shake slow to fast Gobo10

	
	
	 221-225
	 Shake slow to fast Gobo11

	
	
	 226-230
	 Shake slow to fast Gobo12

	
	
	 231-235
	 Shake slow to fast Gobo13

	
	
	 236-240
	 Shake slow to fast Gobo14

	
	
	 241-245
	 Shake slow to fast Gobo15

	
	
	 246-250
	 Shake slow to fast Gobo16

	
	
	 251-255
	 Shake slow to fast Gobo17

	 [ CH5 ]
	 Prism1
	
	

	 
	 
	 0-127
	 None

	 
	 
	 128-191
	 Inert prism1

	 
	 
	 192-255
	 Insert prism2

	 [ CH6 ]
	 Prism1.R
	
	

	 
	 
	 0-127
	 0-360(degree)

	 
	 
	 128-190
	 Rotate forward (fast to slow)

	 
	 
	 191-192
	 Stop

	 
	 
	 193-255
	 Rotate reverse (slow to fast)

	 [ CH7 ]
	 Prism2
	
	

	 
	 
	 0-127
	 None

	 
	 
	 128-191
	 Inert prismA

	 
	 
	 192-255
	 Insert prismB

	 [ CH8 ]
	 Prism2.R
	
	

	 
	 
	 0-127
	 0-360(degree)

	 
	 
	 128-190
	 Rotate forward (fast to slow)

	 
	 
	 191-192
	 Stop

	 
	 
	 193-255
	 Rotate reverse (slow to fast)

	 [ CH9 ]
	 Focus
	 0-255
	 Far to near

	 [ CH10 ]
	 Pan
	 0-255
	 0-540(degree)

	 [ CH11 ]
	 Pan Fine
	 0-255
	 0-2(degree)

	 [ CH12 ]
	 Tilt
	 0-255
	 0-270(degree)

	 [ CH13 ]
	 Tilt Fine
	 0-255
	 0-1(degree)

	 [ CH14 ]
	 PT Spd
	 0-255
	 Fast to slow

	 [ CH15 ]
	PT-Macro
	 0-255
	

	 [ CH16 ]
	 CTO
	 0-255
	

	 [ CH17 ]
	 Frost
	
	

	 
	 
	 0-127
	 None

	 
	 
	 128-191
	 Insert colorful

	 
	 
	 192-255
	 Insert frost

	 [ CH18 ]
	 Lamp
	
	

	
	
	 0-025
	 None

	
	
	 26-100
	 Turn off lamp over 3 second

	
	
	 101-255
	 Turn on over 3 second

	 [ CH19 ]
	 Reset
	
	

	 
	 
	 0-25
	 None

	 
	 
	 26-76
	 Reset Effect motor over 3 second

	 
	 
	 77-127
	 Reset XY motor over 3 second

	 
	 
	 128-255
	 Reset fxiture over 3 second


PAGE  

