Instructions for 5 Retro Lamps With Auxiliary Light


[image: image1.png]



一、Definition of display panel and buttons

[image: image2.png]MENU UP  DOWN ENTER





MENU       UP      DOWN       ENTER
Menu key: select a function

Up key: parameter increment

Down arrow: parameter decrement

Confirm key: Confirm and save
二、MENU

Press the Menu key after power on, and the menu menu will appear in turn; Up or down keys to modify function parameters, confirm keys to save the current function and parameters (with power down memory after saving)。

Menu Menu：
	A001
	 SHAPE  \* MERGEFORMAT 



	A512
	Set the address code, modify it up or down (A001~A512), and save it with the confirm button.

	CH16
	 SHAPE  \* MERGEFORMAT 



	CH16
	Switch up or down the CH04, CH13, CH35, CH44, and four channels, and save with the confirm button. Default CH13

	A007
	 SHAPE  \* MERGEFORMAT 



	N63
	Auxiliary light effect mode, confirm key to save

	S000
	 SHAPE  \* MERGEFORMAT 



	S000
	Modify the running speed of the auxiliary light effect up or down (S000~S255), and save with the confirm button.

	E007
	 SHAPE  \* MERGEFORMAT 



	E21
	Middle main light mode, confirm key to save.

	S000
	 SHAPE  \* MERGEFORMAT 



	S255
	Modify the running speed of the middle main beam (S000~S255) up or down, and save with the confirm button.

	Soud
	 SHAPE  \* MERGEFORMAT 



	Soud
	Voice control mode.

	R255
	 SHAPE  \* MERGEFORMAT 



	R000
	Adjust the brightness of the red light bead (R000~R255) up or down, and press the confirm button to save.

	G255
	 SHAPE  \* MERGEFORMAT 



	G000
	G green light bead dimming, modify the brightness of the green light bead up or down (G000~G255), and confirm to save.

	B255
	 SHAPE  \* MERGEFORMAT 



	B000
	B blue light bead dimming, modify the brightness of the blue light bead up or down (B000~B255), and confirm to save.

	W255
	 SHAPE  \* MERGEFORMAT 



	W000
	W monochrome light brightness, modify the monochrome light brightness up or down (W000~W255), and confirm to save.


三、Master slave control

    Two or more identical lighting fixtures are connected with a three core signal wire using a DMA. All lighting fixtures are set to any address code from A001 to A512, with any one set as the host and the other lighting fixtures as the slave. All slave display screens do not flash; When using the host to adjust the gradient, pulse, jump, voice control, and self-propelled effects, all slaves synchronize the gradient, pulse, jump, voice control, and self-propelled effects。

Special note: 1. Only one host can be set for a group of lighting fixtures. If there are multiple hosts, all lighting fixtures will flash randomly and not synchronize.

2. All lighting fixtures must be functional when the DMX512 console is turned off.
四、Factory settings
    When any address code is A001~A512, press the Menu key for 3 seconds to enter the factory setting. Factory settings mainly include the functions of output power of each circuit of lamps, fan setting mode, setting temperature protection point, and sending parameters. Press the Menu key for 3 seconds to exit any mode set in the factory.
Factory Settings Table：

	R255
	 SHAPE  \* MERGEFORMAT 



	R020
	R red bead current setting, modify the hexagonal red bead current (R020-R255) up or down, and confirm to save.

	G255
	 SHAPE  \* MERGEFORMAT 



	G020
	G green bead current setting, modify the hexagonal green bead current (G020-G255) up or down, and confirm to save.

	B255
	 SHAPE  \* MERGEFORMAT 



	B020
	B blue bead current setting, modify the hexagonal blue bead current (B020-B255) up or down, and confirm to save.

	W255
	 SHAPE  \* MERGEFORMAT 



	W020
	W monochrome lamp current setting, modify the monochrome lamp current setting up or down (W020~W255), and save with the confirm button.

	FAN1
	 SHAPE  \* MERGEFORMAT 



	FAN0
	Fan setting: When the FAN0 light bead is on, start the fan. When FAN1 reaches the set temperature protection point, start the fan. Press the confirm button to save.

	T040
	 SHAPE  \* MERGEFORMAT 



	T-105
	Set the temperature protection point, modify the parameters up or down (40 ℃~75 ℃), and press the confirm button to save.

	Send
	 SHAPE  \* MERGEFORMAT 



	Send
	Send the factory setting parameters of the machine up or down to all other three core signal wires connected in parallel for lighting fixtures; Confirm sending parameters and press the Menu key for 3 seconds


五、DMX512console

After powering on, set the address codes of all lighting fixtures. Connect all lighting fixtures in parallel with a three core signal wire to the DMX512 console, and the address codes will stop flashing, indicating that the DMX512 console signal has been sent to the lighting fixtures. Use the DMX512 console to control relevant functions according to the instructions of each channel.

CH04Channel Description：

	Thoroughfare
	Channel value
	Basic function

	1
	000-255
	R

	2
	000-255
	G

	3
	000-255
	B

	4
	000-255
	W


CH13Channel Description：
	Thoroughfare
	Channel value
	Basic function

	1
	000-255
	Total dimming

	2
	000-255
	RGB strobe

	3
	000-255
	R

	4
	000-255
	G

	5
	000-255
	B

	6
	000-255
	mode

	7
	000-255
	speed

	8
	000-255
	Background color

	9
	000-255
	Background tone light

	10
	000-255
	W strobe

	11
	000-255
	W

	12
	000-255
	mode

	13
	000-255
	speed


CH35Channel Description：
	Thoroughfare
	Channel value
	Basic function

	1
	000-255
	W1

	2
	000-255
	W2

	3
	000-255
	W3

	4
	000-255
	W4

	5
	000-255
	W5

	6
	000-255
	R1

	7
	000-255
	G1

	8
	000-255
	B1

	9
	000-255
	R2

	10
	000-255
	G2

	11
	000-255
	B2

	…
	…
	… …

	33
	000-255
	R10

	34
	000-255
	G10

	35
	000-255
	B10


CH44Channel Description：
	Thoroughfare
	Channel value
	Basic function

	1
	000-255
	Dimming

	2
	000-255
	RGB strobe

	3
	000-255
	R1

	4
	000-255
	G1

	5
	000-255
	B1

	6
	000-255
	R2

	7
	000-255
	G2

	8
	000-255
	B2

	9
	000-255
	R3

	10
	000-255
	G3

	11
	000-255
	B3

	12
	000-255
	R4

	13
	000-255
	G4

	14
	000-255
	B4

	15
	000-255
	R5

	16
	000-255
	G5

	17
	000-255
	B5

	18
	000-255
	R6

	19
	000-255
	G6

	20
	000-255
	B6

	…
	…
	… …

	33
	000-255
	mode

	34
	000-255
	speed

	35
	000-255
	Background color

	36
	000-255
	Background tone light

	37
	000-255
	W strobe

	38
	000-255
	W1

	39
	000-255
	W2

	40
	000-255
	W3

	41
	000-255
	W4

	42
	000-255
	W5

	43
	000-255
	mode

	44
	000-255
	speed


六、Technical parameters:

Voltage: AC100~220V 50/60HZ

Power: 5 60 watt COB beads, 120 5050 beads, 0.3 watt

Control mode: DMX512, self-propelled, master-slave, voice control, with RDM function.

Channels: CH04, CH13, CH35, CH44

Dimming: 32bit 0-100% linear dimming

Features: Dyeing+Explosion+Auxiliary Light

Working temperature: -30 °C~50 °C

Strobe frequency: 1-30Hz

Appearance: Metal, black

Connection method: DMX512 input/output/power input/output.

IP level: IP20

 1 / 6

