WASH 19x40 BEYE LITE

Operation Manual

Parameters:

Voltage: AC100V-240V 50-60Hz

Rated Power: 750W

Light Source: 19x40W RGBW 4in1 LEDs

Color Temperature: 2800K-8500K

Led Lifetime: 50000 hours

Display: Colorful display

Control: DMX512, Auto Run，Sound

Channels:21CH,22CH,35CH,36CH

Pixel Channel:57CH,76CH

Pan: 540°

Tilt: 240°

Strobe：1-25pcs/second, random

Effects: CTO, Wash, Beam , Zoom, RGBW Full Colors Control, Effect Mode(windmill, kaleidoscope), Lens Plate Variable Direction Infinite Rotation

Menu Operation:

| Menu | Secondary menu | Three menus/parameters | Four-level menu |
| --- | --- | --- | --- |
| Address | 001 - 512 |  |  |
|  | Red | 000-255 |  |
|  | Red fine tuning | 000-255 |  |
|  | Green | 000-255 |  |
|  | Green fine tuning | 000-255 |  |
|  | Blue | 000-255 |  |
|  | Blue fine tuning | 000-255 |  |
|  | White | 000-255 |  |
| Aontrol manually | White fine tuning | 000-255 |  |
|  | Colour temperature | 000-255 |  |
|  | Color macro | 000-255 |  |
|  | Stroboscopic | 000-255 |  |
|  | Aiming | 000-255 |  |
|  | Dimming and fine tuning | 000-255 |  |
|  | Level | 000-255 |  |
|  | Horizontal fine adjustment | 000-255 |  |
|  | Perpendicular/vertical | 000-255 |  |
|  | Vertical fine tuning | 000-255 |  |
|  | Function control | 000-255 |  |
|  | Reset | 000-255 |  |
|  | Focus | 000-255 |  |
|  | Focusing rotation | 000-255 |  |
|  | Graphic selection | 000-255 |  |
|  | Graphic speed | 000-255 |  |
|  | Graphic gradient | 000-255 |  |
|  | Graphic red | 000-255 |  |
|  | Graphic green | 000-255 |  |
| Control manually | Graphic blue | 000-255 |  |
|  | Graphic white | 000-255 |  |
|  | Graphic dimming | 000-255 |  |
|  | Background dimming | 000-255 |  |
|  | Graphic conversion | 000-255 |  |
|  | Graphic offset | 000-255 |  |
|  | Foreground stroboscopic | 000-255 |  |
|  | Background stroboscopic | 000-255 |  |
|  | Background selection | 000-255 |  |
| Set up | Channel mode | 21/22/35/36 |  |
|  | Pixel mode | Turn off /RGB/RGBW |  |
|  | Control model | DMX/master control |  |
|  | Horizontal inversion | Close/open |  |
|  | Vertical inversion | Close/open |  |
|  | Color correction | Close/open |  |
|  | Signal processing | Clear/hold |  |
|  | Fan control | Auto/High Speed/Silent |  |
|  | Dimming curve | Exponent 1.5/ Exponent 2.0/ Line/Curve |  |
|  | Dimming frequency | 1500Hz/1000Hz/43700Hz/31000Hz/21400Hz/15100Hz/9600Hz/5600Hz/3700Hz/2400Hz |  |
|  | Language settings | Chinese/English |  |
|  | Display inversion | Normal/reverse |  |
|  | Display mode | 60 seconds/always on |  |
|  | Touch calibration | 000-255 |  |
| Test | Motor reset | Cancel/execute |  |
|  | Factory settings | Cancel/execute |  |
| Factory | Password input | 000 - 255 |  |
|  | Horizontal correction | 000 - 255 |  |
|  | Vertical correction | 000 - 255 |  |
|  | Focusing correction | 000 - 255 |  |
|  | Rotation correction | 000 - 255 |  |
|  | Red correction | 000 - 255 |  |
|  | Green correction | 000 - 255 |  |
|  | Blue correction | 000 - 255 |  |
|  | White correction | 000 - 255 |  |
| Information | Time information | available machine time | xxxH |
|  |  | Total time | xxxH |
|  |  | Boot count | xxxH |
|  | Sensing detection | Horizontal sensing | Error/normal |
|  |  | Vertical sensing | Error/normal |
|  |  | Focusing sensing | Error/normal |
|  |  | temperature sensing | Error/normal |
|  |  | Fan sensing | Error/normal |
|  | Temperature control information | Lamp holder temperature | xxxC |
|  | Software release | Panel version | Vx.xx |
|  |  | Motor version | Vx.xx |
|  |  | Dimming version 1 | Vx.xx |

- DMX Channels

21 Channel

| CH | Function | DMX value | Detailed information |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | Red 0-100% brightness adjustment, from dark to bright. |
| 2 | Red fine tuning | 0-255 | Red fine tuning |
| 3 | Green | 0-255 | Green 0-100% brightness adjustment, from dark to bright. |
| 4 | Green fine tuning | 0-255 | Green fine tuning |
| 5 | Blue | 0-255 | Blue 0-100% brightness adjustment, from dark to bright. |
| 6 | Blue fine tuning | 0-255 | Blue fine tuning |
| 7 | White | 0-255 | White 0-100% brightness adjustment, from dark to bright. |
| 8 | White fine tuning | 0-255 | White fine tuning |
| 9 | Colour temperature | 0-255 | Linear control of color temperature |
| 10 | Macro color | 0-255 | Macro change of color |
| 11 | Stroboscopic | 0-3 | Lights go out |
|  |  | 4-103 | Use the same steps from slow to fast |
|  |  | 104-107 | Light up |
|  |  | 108-207 | From slow to fast |
|  |  | 208-212 | Light up |
|  |  | 213-225 | Low speed random strobe |
|  |  | 226-238 | Medium speed random strobe |
|  |  | 239-251 | High speed random strobe |
|  |  | 252-255 | Light up |
| 12 | Light modulator | 0-255 | 0-100 Linear brightness adjustment |
| 13 | Fine adjustment of dimmer | 0-255 | Fine adjustment of dimmer |
| 14 | X axis control | 0-255 | X axis control |
| 15 | X-axis fine tuning | 0-255 | X-axis fine tuning |
| 16 | Y axis control | 0-255 | Y axis control |
| 17 | Y axis fine tuning | 0-255 | Y axis fine tuning |
| 18 | Function | 0-102 | Empty (no function) |
|  |  | 103-105 | Turn on the pixel function |
|  |  | 106-255 | Empty (no function) |
| 19 | Reset | 0-250 | No function |
|  |  | 251-255 | Wait 5 seconds for automatic reset. |
| 20 | Focus | 0-255 | Focusing control |
| 21 | Rotate | 0-127 | 0 to 60 degrees |
|  |  | 128-191 | Linear Fast to Slow Rotation |
|  |  | 192-255 | Linear Slow to Fast Rotation |

22 Channel

| CH | Function | DMX value | Detailed information |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | Red 0-100% brightness adjustment, from dark to bright. |
| 2 | Red fine tuning | 0-255 | Red fine tuning |
| 3 | Green | 0-255 | Green 0-100% brightness adjustment, from dark to bright. |
| 4 | Green fine tuning | 0-255 | Green fine tuning |
| 5 | Blue | 0-255 | Blue 0-100% brightness adjustment, from dark to bright. |
| 6 | Blue fine tuning | 0-255 | Blue fine tuning |
| 7 | White | 0-255 | White 0-100% brightness adjustment, from dark to bright. |
| 8 | White fine tuning | 0-255 | White fine tuning |
| 9 | Colour temperature | 0-255 | Linear control of color temperature |
| 10 | Macro color | 0-255 | Macro change of color |
| 11 | Stroboscopic | 0-3 | Lights go out |
|  |  | 4-103 | Use the same steps from slow to fast. |
|  |  | 104-107 | Light up |
|  |  | 108-207 | Strongpoint from slow to fast. |
|  |  | 208-212 | Light up |
|  |  | 213-225 | Low speed random strobe |
|  |  | 226-238 | Medium speed random strobe |
|  |  | 239-251 | High speed random strobe |
|  |  | 252-255 | Light up |
| 12 | Light modulator | 0-255 | 0-100 Linear brightness adjustment |
| 13 | Fine adjustment of dimmer | 0-255 | Fine adjustment of dimmer |
| 14 | X axis control | 0-255 | X axis control |
| 15 | X-axis fine tuning | 0-255 | X-axis fine tuning |
| 16 | Y axis control | 0-255 | Y axis control |
| 17 | Y axis fine tuning | 0-255 | Y axis fine tuning |
| 18 | Function | 0-102 | Empty (no function) |
|  |  | 103-105 | Turn on the pixel function |
|  |  | 106-255 | Empty (no function) |
| 19 | Reset | 0-250 | No function |
|  |  | 251-255 | Wait 5 seconds for automatic reset. |
| 20 | Focus | 0-255 | Focusing control |
| 21 | Rotate | 0-127 | 0 to 60 degrees |
|  |  | 128-191 | Linear Fast to Slow Rotation |
|  |  | 192-255 | Linear Slow to Fast Rotation |
| 22 | Frequency | 0-255 | Adjusting of frequency |

35 Channel

| CH | Function | DMX value | Detailed information |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | Red 0-100% brightness adjustment, from dark to bright. |
| 2 | Red fine tuning | 0-255 | Red fine tuning |
| 3 | Green | 0-255 | Green 0-100% brightness adjustment, from dark to bright. |
| 4 | Green fine tuning | 0-255 | Green fine tuning |
| 5 | Blue | 0-255 | Blue 0-100% brightness adjustment, from dark to bright. |
| 6 | Blue fine tuning | 0-255 | Blue fine tuning |
| 7 | White | 0-255 | White 0-100% brightness adjustment, from dark to bright. |
| 8 | White fine tuning | 0-255 | White fine tuning |
| 9 | Colour temperature | 0-255 | Linear control of color temperature |
| 10 | Macro color | 0-255 | Macro change of color |
| 11 | Stroboscopic | 0-3 | Lights go out |
|  |  | 4-103 | Use the same steps from slow to fast. |
|  |  | 104-107 | Light up |
|  |  | 108-207 | Strongpoint from slow to fast. |
|  |  | 208-212 | Light up |
|  |  | 213-225 | Low speed random strobe |
|  |  | 226-238 | Medium speed random strobe |
|  |  | 239-251 | High speed random strobe |
|  |  | 252-255 | Light up |
| 12 | Light modulator | 0-255 | 0-100 Linear brightness adjustment |
| 13 | Fine adjustment of dimmer | 0-255 | Fine adjustment of dimmer |
| 14 | X axis control | 0-255 | X axis control |
| 15 | X-axis fine tuning | 0-255 | X-axis fine tuning |
| 16 | Y axis control | 0-255 | Y axis control |
| 17 | Y axis fine tuning | 0-255 | Y axis fine tuning |
| 18 | Function | 0-102 | Empty (no function) |
|  |  | 103-105 | Turn on the pixel function |
|  |  | 106-255 | Empty (no function) |
| 19 | Reset | 0-250 | No function |
|  |  | 251-255 | Wait 5 seconds for automatic reset. |
| 20 | Focus | 0-255 | Focusing control |
| 21 | Rotate | 0-127 | 0 to 60 degrees |
|  |  | 128-191 | Linear Fast to Slow Rotation |
|  |  | 192-255 | Linear Slow to Fast Rotation |
| 22 | Shape selection | 0-255 | Shape selection |
| 23 | Shape speed | 0-255 | Shape speed |
| 24 | Shape fade | 0-255 | Smooth,fading curve gamma |
| 25 | Shape r | 0-255 | Linear from no light to max brightness |
| 26 | Shape g | 0-255 | Linear from no light to max brightness |
| 27 | Shape b | 0-255 | Linear from no light to max brightness |
| 28 | Shape w | 0-255 | Linear from no light to max brightness |
| 29 | Shape brightness adjustment | 0-255 | Shape dimmer 0-100% |
| 30 | Background brightness adjustment | 0-255 | Background dimmer 0-100% |
| 31 | Shape transition | 0-255 | Shape transition |
| 32 | Shape deviation | 0-255 | Shape deviation |
| 33 | Foreground strobe | 0-255 | Foreground strobe control |
| 34 | Background strobe | 0-255 | Background strobe control |
| 35 | Background select | 000-007 | Wash |
|  |  | 008-255 | Background selection |

36 Channel

| CH | Function | DMX value | Detailed information |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | Red 0-100% brightness adjustment, from dark to bright. |
| 2 | Red fine tuning | 0-255 | Red fine tuning |
| 3 | Green | 0-255 | Green 0-100% brightness adjustment, from dark to bright. |
| 4 | Green fine tuning | 0-255 | Green fine tuning |
| 5 | Blue | 0-255 | Blue 0-100% brightness adjustment, from dark to bright. |
| 6 | Blue fine tuning | 0-255 | Blue fine tuning |
| 7 | White | 0-255 | White 0-100% brightness adjustment, from dark to bright. |
| 8 | White fine tuning | 0-255 | White fine tuning |
| 9 | Colour temperature | 0-255 | Linear control of color temperature |
| 10 | Macro color | 0-255 | Macro change of color |
| 11 | Stroboscopic | 0-3 | Lights go out |
|  |  | 4-103 | Use the same steps from slow to fast. |
|  |  | 104-107 | Light up |
|  |  | 108-207 | Strongpoint from slow to fast. |
|  |  | 208-212 | Light up |
|  |  | 213-225 | Low speed random strobe |
|  |  | 226-238 | Medium speed random strobe |
|  |  | 239-251 | High speed random strobe |
|  |  | 252-255 | Light up |
| 12 | Light modulator | 0-255 | 0-100 Linear brightness adjustment |
| 13 | Fine adjustment of dimmer | 0-255 | Fine adjustment of dimmer |
| 14 | X axis control | 0-255 | X axis control |
| 15 | X-axis fine tuning | 0-255 | X-axis fine tuning |
| 16 | Y axis control | 0-255 | Y axis control |
| 17 | Y axis fine tuning | 0-255 | Y axis fine tuning |
| 18 | Function | 0-102 | Empty (no function) |
|  |  | 103-105 | Turn on the pixel function |
|  |  | 106-255 | Empty (no function) |
| 19 | Reset | 0-250 | No function |
|  |  | 251-255 | Wait 5 seconds for automatic reset. |
| 20 | Focus | 0-255 | Focusing control |
| 21 | Rotate | 0-127 | 0 to 60 degrees |
|  |  | 128-191 | Linear Fast to Slow Rotation |
|  |  | 192-255 | Linear Slow to Fast Rotation |
| 22 | Shape selection | 0-255 | Shape selection |
| 23 | Shape speed | 0-255 | Shape speed |
| 24 | Shape fade | 0-255 | Smooth,fading curve gamma |
| 25 | Shape r | 0-255 | Linear from no light to max brightness |
| 26 | Shape g | 0-255 | Linear from no light to max brightness |
| 27 | Shape b | 0-255 | Linear from no light to max brightness |
| 28 | Shape w | 0-255 | Linear from no light to max brightness |
| 29 | Shape brightness adjustment | 0-255 | Shape dimmer 0-100% |
| 30 | Background brightness adjustment | 0-255 | Background dimmer 0-100% |
| 31 | Shape transition | 0-255 | Shape transition |
| 32 | Shape deviation | 0-255 | Shape deviation |
| 33 | Foreground strobe | 0-255 | Foreground strobe control |
| 34 | Background strobe | 0-255 | Background strobe control |
| 35 | Background select | 000-007 | Wash |
|  |  | 008-255 | Background selection |
| 36 | Frequency | 0-255 | Adjusting of frequency |

57 Channel

| DMX | Function | Bit Value | Function Detailed Description |
| --- | --- | --- | --- |
| 1 | Red 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 2 | Green 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 3 | Blue 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 4 | Red 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 5 | Green 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 6 | Blue 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 7 | Red 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 8 | Green 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 9 | Blue 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| ⁞ | ⁞ | ⁞ | ⁞ |
| ⁞ | ⁞ | ⁞ | ⁞ |
| ⁞ | ⁞ | ⁞ | ⁞ |
| 49 | Red 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 50 | Green 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 51 | Blue 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 52 | Red 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 53 | Green 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 54 | Blue 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 55 | Red 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 56 | Green 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 57 | Blue 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |

76 channel

| DMX | Function | Bit Value | Function Detailed Description |
| --- | --- | --- | --- |
| 1 | Red 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 2 | Green 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 3 | Blue 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 4 | White 1 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 5 | Red 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 6 | Green 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 7 | Blue 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 8 | White 2 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 9 | Red 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 10 | Green 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 11 | Blue 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 12 | White 3 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| ⁞ | ⁞ | ⁞ | ⁞ |
| ⁞ | ⁞ | ⁞ | ⁞ |
| ⁞ | ⁞ | ⁞ | ⁞ |
| ⁞ | ⁞ | ⁞ | ⁞ |
| 65 | Red 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 66 | Green 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 67 | Blue 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 68 | White 17 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 69 | Red 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 70 | Green 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 71 | Blue 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 72 | White 18 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 73 | Red 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 74 | Green 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 75 | Blue 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |
| 76 | White 19 | 0-255 | 0-100% brightness adjustment, from dark to bright. |

Maintenance and Cleaning

The following points have to be considered during the inspection:

- All screws for installing the devices or parts of the device have to be tightly connected and must not be corroded. 2. There must not be any deformations on the housing, color lenses, fixations and installation spots (ceiling, suspension, trussing). 3. Mechanically moved parts must not show any traces of wearing and must not rotate with unbalances. 4. The electric power supply cables must not show any damage, material fatigue or sediments.

Further instructions depending on the installation spot and usage have to be adhered by a skilled installer and any safety problems have to be removed.

In order to make the lights in good condition and extend the life time, we suggest a regular cleaning to the lights.

1)  Clean the inside and outside lens each week to avoid the weakness of the lights due to accumulation of dust.

2)  Clean the fan each week.

3)   A detailed electric check by approved electrical engineer each three month, make sure that the circuit contacts are in good condition,  prevent the poor contact of circuit from overheating.

We recommend a frequent cleaning of the device. Please use a moist, lint- free cloth.  Never use alcohol or solvents.There are no serviceable parts inside the device. Please refer to the instructions under “Installation instructions” .Should you need any spare parts, please order genuine parts from your local dealer.