User manual

AMIXLED WASH 37x40 K25 IP66

Please read the instructions carefully before use

TABLE OF CONTENTS

- Safety Instructions……………………………………………………………………………3-4

- Technical Specifications…………………………………………………………………………4

- Control Menu ……………………………………………………………………………………5

- DMX Channels …………………………………………………………………………………6-16

- Dimension…………………………………………………………………………………………17

- Trouble Shooting…………………………………………………………………………………18

- Fixture Cleaning………………………………………………………… ………………………18

STATEMENT

The product has well capability and intact packing when leave factory. All of  the user should comply with warning item and manual, any misuse cause of the damages are not included in our guarantee, and also can not be responsible  for any malfunction & problem owing to ignore the manual.

1.Safety Instructions

Please keep this User Guide for future consultation. If you sell the unit to another user, be sure that they also receive this instruction booklet.

•Unpack and check carefully there is no transportation damage before using the unit.

•Before operating, ensure that the voltage and frequency of power supply match the power requirements of the unit.

•It’s important to ground the yellow/green conductor to earth in order to avoid electric shock.

•The unit is for indoor use only. Use only in a dry location.

•The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent surfaces. Be sure that no ventilation slots are blocked.

•Disconnect main power before replacement or servicing.

•Make sure there are no flammable materials close to the unit while operating as it is fire hazard.

•Use safety cable when fixes this unit. DO NOT handle the unit by taking its head only, but always by taking its base.

•Maximum ambient temperature is Ta: 40℃. DO NOT operate it where the temperature is higher than this Unit surface temperature may reach up to 85℃. DO NOT touch the housing bare-hand during its operation. Turn off the power and allow about 15 minutes for the unit to cool down before replacing or serving.

•the event of serious operating problem, stop using the unit immediately.Never try to repair the unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please contact the nearest authorized technical assistance center. Always use the same type spare parts.

•DO NOT touch any wire during operation as high voltage might be causing electric shock.

Warning:

•To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

•The housing, the lenses, or the ultraviolet filter must be replaced if they are visibly damaged.

Caution:

•There are no user serviceable parts inside the unit. DO NOT open the housing or attempt any repairs yourself. In the unlikely event your unit may require service, please contact your nearest dealer.

Installation:

•The unit should be mounted via its screw holes on the bracket. Always ensure that the unit is firmly fixed to avoid vibration and slipping while operating. And make sure that the structure to which you are attaching the unit is secure and is able to support a weight of 10 times of the unit’s weight. Also always use a safety cable that can hold 12 times of the weight of the unit when installing the fixture.

•The equipment must be fixed by professionals. And it must be fixed at a place where is out of the touch of people.

2.Technical Specifications

Light source:37*40W RGBW 4in1 LED

Full body Magnesium alloy

Each Led pixel control effect

Wash zoom+ bee eye rotating effect

CTO: 2800K-8500K adjustment

Power consumption: 1500W

Zoom angle:6°-55°

Pan: 540°(16 bit)

Tilt: 270°(16 bit)

Outstanding color Marco effect; Variable strobe effects

Over heat temperature control

0-100% Smooth dimming

DMX Channel: 21/22/35/36/111/148 CH

Control: DMX512/RDM

Display: LCD colorful display

IP Rating: IP66

Dimension:279*530*663mm

NW：31KGS

3.Control Menu

| Main Menu | Second | Third | Fourth |
| --- | --- | --- | --- |
| Dmx Address | 000 - 512 |  |  |
| Fixture | Dmx Mode | 21CH/22CH/35CH/36CH/111CH/148CH |  |
|  | Pixel Mode | Close/111CH/148CH |  |
|  | Run Mode | DMX / Host / Sound |  |
|  | Pan Invert | Close / Open |  |
|  | Tilt Invert | Close / Open |  |
|  | No Dmx Signal | Clear / Hold |  |
|  | Fan Control | Auto / Client / High |  |
|  | Frequency | 600Hz/1200Hz/2000Hz/4000Hz/6000Hz/25KHz |  |
|  | Sound Sensitivity | 00 -- 99 |  |
| Manual | Pan | 000 - 255 |  |
|  | Pan Fine | 000 - 255 |  |
|  | Tilt | 000 - 255 |  |
|  | Tilt Fine | 000 - 255 |  |
|  | ⁞ | ⁞ |  |
| Information | Time | Current Time | xxxH |
|  |  | Total Time | xxxH |
|  |  | Power Count | xxxx |
|  | Sensor | All Sensors are ok! / Pan、Tilt、Zoom..... |  |
|  | Temperature | Head Temp | xxxC |
|  | Software Version | Panel | Vx.xx |
|  |  | Motor | Vx.xx |
|  |  | Dimmer1 | Vx.xx |
| Factory | Password | 000 - 255 |  |
|  | Pan | 000 - 255 |  |
|  | Tilt | 000 - 255 |  |
|  | Zoom | 000 - 255 |  |
|  | Zoom Rotation | 000 - 255 |  |
|  | Red | 000 - 255 |  |
|  | Green | 000 - 255 |  |
|  | Blue | 000 - 255 |  |
|  | White | 000 - 255 |  |
| Fixture Reset | Motor Reset | Cancel / Run |  |
|  | Factory | Cancel / Run |  |
| Display | Language | CH / EN |  |
|  | Display Flip | Normal / Reverse |  |
|  | Display Mode | 60s / Show |  |

4.DMX Channels

21CH Mode

| Channel | Function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 2 | Red fine | 0-255 | Red fine |
| 3 | Green | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 4 | Green fine | 0-255 | Green fine |
| 5 | Blue | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 6 | Blue fine | 0-255 | Blue fine |
| 7 | White | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 8 | White fine | 0-255 | White fine |
| 9 | CTO | 0-255 | 8500K-2800K Color temperature linear control |
| 10 | Macro color | 0-255 | Color Macro Variations |
| 11 | Strobe | 0 - 3 | Close |
|  |  | 4 - 103 | Regular flashing, speed changes from slow to fast |
|  |  | 104 - 107 | Open |
|  |  | 108 - 207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208 - 212 | Open |
|  |  | 213 - 225 | Random slow strobe |
|  |  | 226 - 238 | Random medium speed strobe |
|  |  | 239 - 251 | Random fast strobe |
|  |  | 252 - 255 | Open |
| 12 | Dimmer | 0-255 | 0-100Linear brightness adjustment |
| 13 | Dimmer fine | 0-255 | Overall dimming fine adjustment |
| 14 | Pan | 0-255 | X Axis Control |
| 15 | Pan fine | 0-255 | X Axis fine |
| 16 | Tilt | 0-255 | Y Axis Control |
| 17 | Tilt fine | 0-255 | Y Axis fine |
| 18 | Function | 0-255 | Channel Function |
| 19 | Reset | 0-199 | Null |
|  |  | 200-205 | Wait 5 seconds for automatic reset |
|  |  | 206-255 | Null |
| 20 | Zoom | 0-255 | Focus control |
| 21 | Zoom Rotation | 0-255 | Rotation Control |

22CH Mode

| Channel | Function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 2 | Red fine | 0-255 | Red fine |
| 3 | Green | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 4 | Green fine | 0-255 | Green fine |
| 5 | Blue | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 6 | Blue fine | 0-255 | Blue fine |
| 7 | White | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 8 | White fine | 0-255 | White fine |
| 9 | CTO | 0-255 | 8500K-2800KColor temperature linear control |
| 10 | Macro color | 0-255 | Color Macro Variations |
| 11 | Strobe | 0 - 3 | Close |
|  |  | 4 - 103 | Regular flashing, speed changes from slow to fast |
|  |  | 104 - 107 | Open |
|  |  | 108 - 207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208 - 212 | Open |
|  |  | 213 - 225 | Random slow strobe |
|  |  | 226 - 238 | Random medium speed strobe |
|  |  | 239 - 251 | Random fast strobe |
|  |  | 252 - 255 | Open |
| 12 | Dimmer | 0-255 | 0-100 Linear brightness adjustment |
| 13 | Dimmer fine | 0-255 | Total dimming fine adjustment |
| 14 | Pan | 0-255 | X Axis Control |
| 15 | Pan fine | 0-255 | X Axis fine |
| 16 | Tilt | 0-255 | Y Axis Control |
| 17 | Tilt fine | 0-255 | Y Axis fine |
| 18 | Function | 0-255 | Channel Function |
| 19 | Reset | 0-199 | Null |
|  |  | 200-205 | Wait 5 seconds for automatic reset |
|  |  | 206-255 | Null |
| 20 | Zoom | 0-255 | Focus control |
| 21 | Zoom Rotation | 0-255 | Rotation Control |
| 22 | Frequency | 0-255 | Frequency selection |

35CH Mode

| Channel | Function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 2 | Red fine | 0-255 | Red fine |
| 3 | Green | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 4 | Green fine | 0-255 | Green fine |
| 5 | Blue | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 6 | Blue fine | 0-255 | Blue fine |
| 7 | White | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 8 | White fine | 0-255 | White fine |
| 9 | CTO | 0-255 | 8500K-2800KColor temperature linear control |
| 10 | Macro color | 0-255 | Color Macro Variations |
| 11 | Strobe | 0 - 3 | Close |
|  |  | 4 - 103 | Regular flashing, speed changes from slow to fast |
|  |  | 104 - 107 | Open |
|  |  | 108 - 207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208 - 212 | Open |
|  |  | 213 - 225 | Random slow strobe |
|  |  | 226 - 238 | Random medium speed strobe |
|  |  | 239 - 251 | Random fast strobe |
|  |  | 252 - 255 | Open |
| 12 | Dimmer | 0-255 | 0-100Linear brightness adjustment |
| 13 | Dimmer fine | 0-255 | Dimmer fine |
| 14 | Pan | 0-255 | X Axis Control |
| 15 | Pan fine | 0-255 | X Axis fine |
| 16 | Tilt | 0-255 | Y Axis Control |
| 17 | Tilt fine | 0-255 | Y Axis fine |
| 18 | Function | 0-102 | No Function |
|  |  | 103-105 | Turn on pixel mode |
|  |  | 106-255 | No Function |
| 19 | Reset | 0-199 | Null |
|  |  | 200-205 | Wait 5 seconds for automatic reset |
|  |  | 206-255 | No function |
| 20 | Zoom | 0-255 | Focus control |
| 21 | Zoom Rotation | 0-255 | Rotation Control |
| 22 | Shape Selection | 0-255 | Shape Selection |
| 23 | Shape Fade | 0-255 | Smooth gradient control |
| 24 | Shape Speed | 0-255 | Shape speed control |
| 25 | Shape Red | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 26 | Shape Green | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 27 | Shape Blue | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 28 | Shape White | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 29 | Shape Dimmer | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 30 | Background Dimmer | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 31 | Shape Transition | 0-255 | Shape Transition |
| 32 | Shape Offset | 0-255 | Shape Offset |
| 33 | Foreground Strobe | 0-4 | Close |
|  |  | 5-103 | Regular flashing, speed changes from slow to fast |
|  |  | 104-107 | Open |
|  |  | 108-207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208-255 | Open |
| 34 | Background Strobe | 0-4 | Close |
|  |  | 5-103 | Regular flashing, speed changes from slow to fast |
|  |  | 104-107 | Open |
|  |  | 108-207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208-255 | Open |
| 35 | Background Select | 0-255 | Rotation Control |

36CH Mode

| Channel | Function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | Red | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 2 | Red fine | 0-255 | Red fine |
| 3 | Green | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 4 | Green fine | 0-255 | Green fine |
| 5 | Blue | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 6 | Blue fine | 0-255 | Blue fine |
| 7 | White | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 8 | White fine | 0-255 | White fine |
| 9 | CTO | 0-255 | 8500K-2800KColor temperature linear control |
| 10 | Macro color | 0-255 | Color Macro Variations |
| 11 | Strobe | 0 - 3 | Close |
|  |  | 4 - 103 | Regular flashing, speed changes from slow to fast |
|  |  | 104 - 107 | Open |
|  |  | 108 - 207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208 - 212 | Open |
|  |  | 213 - 225 | Random slow strobe |
|  |  | 226 - 238 | Random medium speed strobe |
|  |  | 239 - 251 | Random fast strobe |
|  |  | 252 - 255 | Open |
| 12 | Dimmer | 0-255 | 0-100Linear brightness adjustment |
| 13 | Dimmer fine | 0-255 | Dimmer fine |
| 14 | Pan | 0-255 | X Axis Control |
| 15 | Pan fine | 0-255 | X Axis fine |
| 16 | Tilt | 0-255 | Y Axis Control |
| 17 | Tilt fine | 0-255 | Y Axis fine |
| 18 | Function | 0-102 | No Function |
|  |  | 103-105 | Turn on pixel mode |
|  |  | 106-255 | No Function |
| 19 | Reset | 0-199 | Null |
|  |  | 200-205 | Wait 5 seconds for automatic reset |
|  |  | 206-255 | No function |
| 20 | Zoom | 0-255 | Focus control |
| 21 | Zoom Rotation | 0-255 | Rotation Control |
| 22 | Shape Selection | 0-255 | Shape Selection |
| 23 | Shape Fade | 0-255 | Smooth gradient control |
| 24 | Shape Speed | 0-255 | Shape speed control |
| 25 | Shape Red | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 26 | Shape Green | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 27 | Shape Blue | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 28 | Shape White | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 29 | Shape Dimmer | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 30 | Background Dimmer | 0-255 | 0-100%Brightness adjustment, from dark to bright |
| 31 | Shape Transition | 0-255 | Shape Transition |
| 32 | Shape Offset | 0-255 | Shape Offset |
| 33 | Foreground Strobe | 0-4 | Close |
|  |  | 5-103 | Regular flashing, speed changes from slow to fast |
|  |  | 104-107 | Open |
|  |  | 108-207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208-255 | Open |
| 34 | Background Strobe | 0-4 | Close |
|  |  | 5-103 | Regular flashing, speed changes from slow to fast |
|  |  | 104-107 | Open |
|  |  | 108-207 | Pulse strobe, speed changes from slow to fast |
|  |  | 208-255 | Open |
| 35 | Background Select | 0-255 | Rotation Control |
| 36 | Frequency | 0-255 | Frequency selection |

111CH Mode

| Channel | Function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | Red 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 2 | Green 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 3 | Blue 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 4 | Red 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 5 | Green 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 6 | Blue 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 7 | Red 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 8 | Green 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 9 | Blue 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 10 | Red 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 11 | Green 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 12 | Blue 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 13 | Red 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 14 | Green 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 15 | Blue 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 16 | Red 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 17 | Green 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 18 | Blue 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 19 | Red 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 20 | Green 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 21 | Blue 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 22 | Red 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 23 | Green 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 24 | Blue 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 25 | Red 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 26 | Green 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 27 | Blue 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 28 | Red 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 29 | Green 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 30 | Blue 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 31 | Red 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 32 | Green 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 33 | Blue 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 34 | Red 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 35 | Green 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 36 | Blue 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 37 | Red 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 38 | Green 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 39 | Blue 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 40 | Red 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 41 | Green 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 42 | Blue 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 43 | Red 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 44 | Green 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 45 | Blue 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 46 | Red 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 47 | Green 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 48 | Blue 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 49 | Red 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 50 | Green 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 51 | Blue 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 52 | Red 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 53 | Green 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 54 | Blue 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 55 | Red 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 56 | Green 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 57 | Blue 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 58 | Red 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 59 | Green 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 60 | Blue 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 61 | Red 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 62 | Green 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 63 | Blue 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 64 | Red 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 65 | Green 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 66 | Blue 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 67 | Red 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 68 | Green 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 69 | Blue 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 70 | Red 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 71 | Green 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 72 | Blue 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 73 | Red 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 74 | Green 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 75 | Blue 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 76 | Red 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 77 | Green 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 78 | Blue 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 79 | Red 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 80 | Green 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 81 | Blue 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 82 | Red 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 83 | Green 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 84 | Blue 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 85 | Red 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 86 | Green 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 87 | Blue 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 88 | Red 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 89 | Green 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 90 | Blue 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 91 | Red 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 92 | Green 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 93 | Blue 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 94 | Red 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 95 | Green 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 96 | Blue 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 97 | Red 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 98 | Green 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 99 | Blue 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 100 | Red 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 101 | Green 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 102 | Blue 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 103 | Red 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 104 | Green 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 105 | Blue 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 106 | Red 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 107 | Green 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 108 | Blue 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 109 | Red 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 110 | Green 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 111 | Blue 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |

148CH Mode

| Channel | Function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | Red 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 2 | Green 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 3 | Blue 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 4 | White 1 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 5 | Red 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 6 | Green 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 7 | Blue 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 8 | White 2 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 9 | Red 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 10 | Green 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 11 | Blue 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 12 | White 3 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 13 | Red 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 14 | Green 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 15 | Blue 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 16 | White 4 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 17 | Red 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 18 | Green 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 19 | Blue 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 20 | White 5 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 21 | Red 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 22 | Green 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 23 | Blue 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 24 | White 6 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 25 | Red 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 26 | Green 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 27 | Blue 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 28 | White 7 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 29 | Red 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 30 | Green 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 31 | Blue 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 32 | White 8 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 33 | Red 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 34 | Green 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 35 | Blue 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 36 | White 9 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 37 | Red 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 38 | Green 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 39 | Blue 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 40 | White 10 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 41 | Red 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 42 | Green 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 43 | Blue 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 44 | White 11 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 45 | Red 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 46 | Green 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 47 | Blue 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 48 | White 12 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 49 | Red 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 50 | Green 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 51 | Blue 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 52 | White 13 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 53 | Red 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 54 | Green 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 55 | Blue 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 56 | White 14 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 57 | Red 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 58 | Green 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 59 | Blue 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 60 | White 15 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 61 | Red 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 62 | Green 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 63 | Blue 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 64 | White 16 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 65 | Red 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 66 | Green 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 67 | Blue 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 68 | White 17 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 69 | Red 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 70 | Green 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 71 | Blue 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 72 | White 18 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 73 | Red 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 74 | Green 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 75 | Blue 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 76 | White 19 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 77 | Red 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 78 | Green 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 79 | Blue 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 80 | White 20 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 81 | Red 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 82 | Green 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 83 | Blue 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 84 | White 21 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 85 | Red 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 86 | Green 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 87 | Blue 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 88 | White 22 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 89 | Red 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 90 | Green 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 91 | Blue 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 92 | White 23 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 93 | Red 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 94 | Green 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 95 | Blue 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 96 | White 24 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 97 | Red 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 98 | Green 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 99 | Blue 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 100 | White 25 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 101 | Red 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 102 | Green 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 103 | Blue 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 104 | White 26 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 105 | Red 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 106 | Green 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 107 | Blue 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 108 | White 27 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 109 | Red 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 110 | Green 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 111 | Blue 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 112 | White 28 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 113 | Red 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 114 | Green 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 115 | Blue 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 116 | White 29 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 117 | Red 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 118 | Green 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 119 | Blue 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 120 | White 30 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 121 | Red 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 122 | Green 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 123 | Blue 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 124 | White 31 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 125 | Red 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 126 | Green 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 127 | Blue 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 128 | White 32 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 129 | Red 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 130 | Green 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 131 | Blue 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 132 | White 33 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 133 | Red 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 134 | Green 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 135 | Blue 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 136 | White 34 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 137 | Red 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 138 | Green 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 139 | Blue 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 140 | White 35 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 141 | Red 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 142 | Green 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 143 | Blue 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 144 | White 36 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 145 | Red 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 146 | Green 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 147 | Blue 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |
| 148 | White 37 | 0-255 | 0-100% brightness adjustment, from dark to bright |

5.Trouble Shooting

6.Trouble Shooting

Following are a few common problems that may occur during operation. Here are some suggestions for easy troubleshooting:

A. The unit does not work, no light and the fan does not work

1. Check the connection of power and main fuse.

2. Measure the mains voltage on the main connector.

3. Check the power on LED.

B. Not responding to DMX controller

1. DMX LED should be on. If not, check DMX connectors, cables to see if link properly.

2. If the DMX LED is on and no response to the channel, check the address settings and DMX polarity.

3. If you have intermittent DMX signal problems, check the pins on connectors or on

PCB of the unit or the previous one.

4. Try to use another DMX controller.

5. Check if the DMX cables run near or run alongside to high voltage cables that may cause damage or interference to DMX interface circuit.

C. One of the channels is not working well

1. The stepper motor might be damaged or the cable connected to the PCB is broken.

2. The motor’s drive IC on the PCB might be out of condition

7.Fixture Cleaning

The cleaning of internal and external optical lenses and/or mirrors must be carried out periodically to optimize light output. Cleaning frequency depends on the environment in which the fixture operates: damp, smoky or particularly dirty surrounding can cause greater accumulation of dirt on the unit’s optics.

 Clean with soft cloth using normal glass cleaning fluid.

 Always dry the parts carefully.

 Clean the external optics at least every 20 days. Clean the internal optics at least every 30/60 days.